| SR. No. |  |  |  |  |  |  | SOR 2016 |  |  |  | SPECIFICATION |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A . |  |  |  |  |  |  |  |  |  |  | CIVIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  | 15.8 |  |  |  | Removing mortar form bricks and cleaning bricks including stacking within a lead of 50m (stacks of cleaned bricks shall be measured): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 15.8.3 |  |  |  | From brick work in cement mortar |  |  |  |  |  | Cum |  |  |  | 13 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  | 15.2 |  |  |  | Demolishing cement concrete including disposal of material within 50 metre lead: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 15.2.1 |  |  |  | 1:3:6 or leaner mix |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  | 15.56 |  |  |  | Dismantling old plaster or skirting raking out joints and cleaning the surface for plaster including disposal of rubbish to the dumping ground within 50 metres lead. |  |  |  |  |  | Sqm. |  |  |  | 250 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | 15.12 |  |  |  | Demolition of existing doors/ windows/Rolling Shutter & removal of debries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 15.12.1 |  |  |  | Of area 3 sqm and below |  |  |  |  |  | Sqm. |  |  |  | 16 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  | N.S. |  |  |  | Demolition of existing sanitary/plumbing fixtures & removal of debries |  |  |  |  |  | L.S. |  |  |  | 1 |  |  |  |  |  |  |  |  |
| 6 a) |  |  |  |  |  |  | N.S. |  |  |  | Chase cutting at floor |  |  |  |  |  | Rmt |  |  |  | 156 |  |  |  |  |  |  |  |  |
| b) |  |  |  |  |  |  | N.S. |  |  |  | Chase cutting at wall |  |  |  |  |  | Rmt |  |  |  | 301 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | N.S. |  |  |  | Providing Pest control and Anti-termite treatment by appointing a specialized agency as per the specifications mentioned by the Bureau of Indian Standards and Agencies specifications (whichever is higher) for general civil, plumbing / drainage and timber / carpentry works, gypsum related work, including a 5-year guarantee under suitable undertaking on stamp paper etc, complete as directed. (Mode of measurement to be on carpet-area of floor and not the area of surface treated) With chlopyriphons EC 20 % with 1 % concentration. |  |  |  |  |  | Sqm. |  |  |  | 1,300.00 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  | 6.1.14A +6.3A |  |  |  | Extra for brick work in superstructure above plinth level upto floor V ( cement mortar 1:6 ( 1 cement :6 coarse sand ) |  |  |  |  |  | Cum |  |  |  | 6.80 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | 6.18.4A+6.19A |  |  |  | Extra for Half brick masonry in superstructure above plinth level upto floor V level ( cement mortar 1:4) |  |  |  |  |  | Sqm. |  |  |  | 54 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  | 13.11.2 |  |  |  | Cement plaster in course sand 12mm cement plaster of mix : ( 12:4 ( 1ciment :4: sand ) |  |  |  |  |  | Sqm. |  |  |  | 200 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  | 5.3 |  |  |  | Reinforced cement concrete work in beams, suspended floors, roofs having slope upto 15, landings, balconiec, shelves, chajjas, lintels, bands, plain window sills, staircases and spiral stair cases upto floor five level excluding the cost of centering, shuttering, finishing and reinforcement with 1:2:4(1 cement:2 coarse sand:4 graded stone aggregate 20 mm nominal size). |  |  |  |  |  | Cum |  |  |  | 2 |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | 5.22 |  |  |  | Reinforcement for R.C.C work including straightening, cutting, bending, placing in position and binding all complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 5.22.7A |  |  |  | Thermo-Mechanically treated bars TMTC - 500- 8 mm dia |  |  |  |  |  | kg |  |  |  | 350.00 |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  | 5.9 |  |  |  | Centring and shuttering including strutting, propping etc. and removal of form for: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 5.9 .5 |  |  |  | Lintels, beams, plinth beams, girders, bressumers and cantilevers. |  |  |  |  |  | Sqm |  |  |  | 19.00 |  |  |  |  |  |  |  |  |
| B. |  |  |  |  |  |  |  |  |  |  | FLOORING & CLADDING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  | 4.1 |  |  |  | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering-all work upto plinth level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 4.1.3 |  |  |  | 1:2:4(1 Cement: 2 coarse sand:4 graded stone aggregate 20 mm nominal size) |  |  |  |  |  | Cum |  |  |  | 11.00 |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | 11.4 |  |  |  | IPS Flooring52 mm thick cement concrete flooring with metallic concrete hardener topping under layer 40 mm thick cement concrete 1:2:4(1cement:2 coarse sand:4 graded stone aggregate 20 mm nominal size) and top layer 12 mm thick metallic concrete hardener consisting of mix 1:2 (1 cement hardener mix:2 stone aggregate 6 mm nominal size) by volume @ 2 liter per 50 kg of cement or as per manufacture specification. This includes cost of cement slurry, etc. but excluding the cost of nosing of steps etc. complete. |  |  |  |  |  | Sqm. |  |  |  | 160.00 |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  | 11.46 .2 |  |  |  | Providing and laying Vitrified tiles in different sizes (thickness to be specified by manufacturer), with water absorption less than 0.08 % and conforming to I.S. 15622, of approved make, in all colors & shade, in skirting, riser of steps, over 12 mm thick bed of cement mortar 1:3 (1 cement: 3 coarse sand), including grouting the joint with white cement & matching pigments etc. complete. |  |  |  |  |  | Sqm. |  |  |  | 986 |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | 11.38 |  |  |  | Providing and laying Ceramic glazed floor tiles 300 x 300 mm (thickness to be specified by the manufacturer) of 1st quality conforming to IS: 15622 of approved make in colors such shades except white, Ivory, Grey, Fume Red, Brown, laid on 20 mm thick Cement mortar 1:4 (1 cement: 4 coarse sand) including pointing the joints with white cement and matching pigment etc., complete. |  |  |  |  |  | Sqm. |  |  |  | 73 |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  | 11.47 |  |  |  | Providing and laying Vitrified tiles in different sizes (thickness to be specified by the manufacturer), with water absorption less than 0.08% and conforming to IS: 15622, of approved brand & manufacturer, in all colors and shade, in skirting, riser of steps, laid with cement based high polymer modified quick set tile adhesive (water based) conforming to IS: 15477, in average 6 mm thickness, including grouting of joints (Payment for grouting of joints to be made separately). |  |  |  |  |  | Sqm. |  |  |  | 44 |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  | N.S. |  |  |  | Providing and laying of floor carpet tile 500mm x 500mm of nylon yarn, 7-8mm thick of approved make and shade pasted on smooth dust-free surface with adhesive as per manufacturer's specification. Rate includes the plastic cover till final handover of site. |  |  |  |  |  | Sqm. |  |  |  | 120.00 |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  | N.S. |  |  |  | Wooden Skirting Providing and fixing skirting of prelaminated with (one side decorative other side balancing lamination) fiat pressed, 3 layer and graded particle board (medium density) (Grade 1, type2) conforming to IS: 12823. With necessary fixing arrangements and screws including drilling necessary holes for rawl plugs etr. and priming coat on unexposed surface complete ( 18 mm thick |  |  |  |  |  | RM |  |  |  | 26 |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  | N.S. |  |  |  | False Flooring (Supply and fixing of 600mm x 600mm false floor cementicous concrete infill false flooring system (ESRGS) & inclusive of a top finish of high pressure anti-static laminate of factory finish covered with anti-static PVC edge beading. Should be an ISO certified product. The system should be able to withstand a minimum point load of 300 Kg per sq.cm & the pedestal assembly should be able to withstand a uniformly distributed load of 800 Kg per sq.m. Top plate & Bottom plate of 0.7mm steel width. Pedestal pipe should be yellow zinc plated 450mm high. ) |  |  |  |  |  | Sqm. |  |  |  | 10.00 |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  | 8.2. |  |  |  | Granite Flooring/window sill:- ( Providing and fixing 18mm thick mirror polished, machine cut for kitchen platforms, vanity counters facias and similar locations of required size of approved shade, color and texture laid over 20 mm thick base cement mortar 1:4 (1 cement :4 coarse sand) with joints treated with white cement, mixed with matching pigament-epoxy touch ups. including rubbing, curing etc. complete at all Ibevels.) |  |  |  |  |  | Sqm. |  |  |  |  |  |  |  |  |  |  |  |  |
| a) |  |  |  |  |  |  | 8.2.2.2 |  |  |  | Granite of any color and shade ( Area of slab over 0.50 sqm ) |  |  |  |  |  | sqm |  |  |  | 90.00 |  |  |  |  |  |  |  |  |
| b) |  |  |  |  |  |  | 8.2.2.2 |  |  |  | Granite Cladding:- Granite of any color and shade ( Area of slab over 0.50 sqm.) |  |  |  |  |  | Sqm. |  |  |  | 60.00 |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  | 8.3 |  |  |  | Providing edge moulding to 18 mm thick marble stone counters, vanities etc., including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 8.3.2 |  |  |  | Granite work |  |  |  |  |  | M |  |  |  | 160 |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  | 11.36 |  |  |  | Providing and fixing 1st quality ceramic glazed wall tiles conforming to IS:15622 ( thickness to be specified by the manufacture) of approved make in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-chage in skirting, risers of steps and dados over 12mm thick bed of Cement Mortar 1:3 (1 Cement:3 Coarse sand ) and jointing with grey cement slurry @ 3.3kg per sqm including pointing in white cement mixed with pigment of matching shade complete. |  |  |  |  |  | Sqm |  |  |  | 190.00 |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  | N.S. |  |  |  | Pre-Laminated Wooden Flooring :-Supply & installation of pre-laminated plank flooring, 8 mm. thick over 2 mm. thick underlay foam completed with all accessories as approved by Architect. |  |  |  |  |  | Sqm. |  |  |  | 46 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | C. |  |  |  | FALSE CEILING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  | 12.45 |  |  |  | Plain Gypsum False Ceiling :-Providing and fixing gypsum false ceiling of India Gypsum make which includes G.I. perimeter channels of size 0.55mm thick (having one flange of 20mm and another flange of 30mm and a web of 27mm) along with perimeter of ceiling, screw fixed to brick wall / partition with the help of nylon sleeves and screws at 610mm centres. Then suspending G.I. intermediate channels of size 45mm (0.9mm thick with two flanges of 15mm each) from the soffit at 1220mm centres with ceiling angle of width 25mm X 10mm X0.55mm thick fixed to soffit with G.I. cleat and steel expansion fasteners. Ceiling section of 0.55mm thick having knurled web of 51.5mm and two flanges of 26mm each with lips of 10.5mm are then fixed to the intermediate channel with the help of connecting clips and in direction perpendicular to the intermediate channel at 457mm centres. 12.5mm tapered edge gypboard (conforming to IS 2095-1982) is then screw fixed to ceiling section with 25mm dry wall Philips screws at 230mm centres. Screw fixing is done mechanically with drilling machine with suitable attachment. Finally tapered edges of the gypboards are to be jointed and finished so as to have a flush look which includes filling and finishing with jointing compound, joint paper tape and two coats of dry wall top coat suitable for gypboard (as per recommended practice of BPB India gypsum ) etc. complete as per the recommended practices of India Gypsum. Rate to be included all kinds of profiles and cut-outs required for light fixtures, speakers, smoke detectors, trap doors and AC grill in the ceiling. All steel sections to be marked with "GYPSTEEL" which is a registered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 12.45.4 |  |  |  | Fully Perforated Gypsum Plaster Board of size 1200 x 2400x12.5 mm having approx. 15 % perforated area with perforation size and pattern as approved by the Engineer in charge and as per manufacturer’s specification, with all 4 side tapered and backed by acoustical tissue with NRC value not less than 0.60 |  |  |  |  |  | Sqm. |  |  |  | 500 |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  | 12.52 |  |  |  | Providing and fixing tiled false ceiling of approved materials of size 595x595 mm in true horizontal level, suspended on inter locking metal grid of hot dipped galvanized steel sections ( galvanized @ 120 grams/ sqm., both side inclusive) consisting of main “T” runner with suitably spaced joints to get required length and of size 24x38 mm made from 0.30 mm thick (minimum) sheet, spaced at 1200 mm center to center and cross “T” of size 24x25 mm made of 0.30 mm thick (minimum) sheet, 1200 mm long spaced between main “T” at 600 mm center to center to form a grid of 1200x600 mm and secondary cross “T” of length 600 mm and size 24x25 mm made of 0.30 mm thick (minimum) sheet to be interlocked at middle of the 1200x600 mm panel to form grids of 600x600 mm and wall angle of size 24x24x0.3 mm and laying false ceiling tiles of approved texture in the grid including, required cutting/making, opening for services like diffusers, grills, light fittings, fixtures, smoke detectors etc. Main “T” runners to be suspended from ceiling using GI slotted cleats of size 27 x 37 x 25 x1.6 mm fixed to ceiling with 12.5 mm dia and 50 mm long dash fasteners, 4 mm GI adjustable rods with galvanised butterfly level clips of size 85 x 30 x 0.8 mm spaced at 1200 mm center to center along main T, bottom exposed width of 24 mm of all T-sections shall be pre-painted with polyester paint, all complete for all heights as per specifications, drawings and as directed by Engineer-in-charge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 12.52.1 |  |  |  | GI Metal Ceiling Lay in plain Tegular edge Global white color tiles of size 595x595 mm, and 0.5 mm thick with 8 mm drop; made of G I sheet having galvanizing of 100 gms/sqm (both sides inclusive) and electro statically polyester powder coated of thickness 60 microns (minimum), including factory painted after bending. |  |  |  |  |  | Sqm |  |  |  | 900.00 |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  | 12.58 |  |  |  | Providing and fixing tiled false ceiling of approved materials of size 595x595 mm in true horizontal level, suspended on interlocking metal grid of hot dipped galvanized steel sections (galvanized @ 120 grams/ sqm, both side inclusive) consisting of main “T” runner with suitably spaced joints to get required length and of size 24x38 mm made from 0.30 mm thick (minimum) sheet, spaced at 1200 mm center to center and cross “T” of size 24x25 mm made of 0.30 mm thick (minimum) sheet, 1200 mm long spaced between main “T” at 600 mm center to center to form a grid of 1200x600 mm and secondary cross “T” of length 600 mm and size 24x25 mm made of 0.30 mm thick (minimum) sheet to be interlocked at middle of the 1200x600 mm panel to form grids of 600x600 mm and wall angle of size24 x24x 0.3 mm and laying false ceiling tiles of approved texture in the grid including, required cutting/making, opening for services like diffusers, grills, light fittings, fixtures, smoke detectors etc. Main “T” runners to be suspended from ceiling using GI slotted cleats of size 27 x 37 x 25 x1.6 mm fixed to ceiling with 12.5 mm dia and 50 mm long dash fasteners, 4 mm GI adjustable rods with galvanized butterfly level clips of size 85 x 30 x 0.8 mm spaced at 1200 mm center to center along main T, bottom exposed width of 24 mm of all T-sections shall be pre-painted with polyester paint, all complete for all heights as per specifications, drawings and as directed by Engineer-in-charge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 12.58.1 |  |  |  | 8 mm thick fully perforated calcium silicate board made with Calcareous & Siliceous materials reinforced with cellulose fiber manufactured through autoclaving process to give stable crystalline structure with minimum compressive strength 225 kg/ sq. cm, bending strength 100 kg/sq. cm , of size 595x595mm, having perforation of dia. 10 mm with minimum perforated area 18 % with non-woven tissue on the back side, having an NRC ( Noise Reduction Coefficient) of 0.85, with 50 mm thick Rockwool of 48 kg /cum backing. |  |  |  |  |  | sqm |  |  |  | 200 |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  | N.S. |  |  |  | Rafter Ceiling :- Supply & installation of Wooden rafter ceiling with 12mm BWR grade Plywood with Textured Laminate finish of approved make of section 150x150mm @200mm c/c of approved design with necessary ss hanging support arrangement above ceiling . All complete as per direction of EIC/Architect. Rate will be calculated on the effective ceiling area. |  |  |  |  |  | Sqm.. |  |  |  | 12.00 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 4 |  |  |  | PARTITION & PANELLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 ( a) |  |  |  |  |  |  | N.S. |  |  |  | Full Height Gypsum Partition :- Providing and fixing 100mm thick gypsum partition which includes two layers of tapered edge 12.5mm thick gypboard (conforming to IS 2095-1982 & 2542-1981) screw fixed with 25mm dry wall screw for first layer and 35mm for second layer at 300mm centres to either side of 48mm studs (0.55mm thick having one flange of 34mm and another flange of 36mm made of GI steel) placed at 610mm centre to centre in 50mm floor and ceiling channel (0.55mm thick having equal flanges of 32mm made of GI steel) with joints staggered on each layer to avoid through joints. Finally square and tapered edges of the boards are to be jointed and finished so as to have a flush look which includes filling and finishing with jointing compound, joint paper tape and two coats of drywall top coat suitable for gypboard (as per recommended practice of India Gypsum or equivalent ) . Rate shall be inclusive USG make gypsum edge guard on edges of the partition and finished as per manufacturer's specifications. Necessary cut- outs shall be provided for taking of supply and return air ducts / openings which shall be neatly finished with PU sealant of HILTI make after fixing of ducts thereby leaving no gaps. The partition should be finished so as to take on wall paper / paint directly by applying suitable base coats etc. all complete. Measurement shall |  |  |  |  |  | Sqm. |  |  |  | 382 |  |  |  |  |  |  |  |  |
| ( b) |  |  |  |  |  |  | N.S. |  |  |  | Above ceiling gypsum |  |  |  |  |  | Sqm. |  |  |  | 67 |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  | N.S. |  |  |  | Rock Wool Insulation :-Providing and filling rock wool insulation of density 24 kg per cum wrapped in RP tissues, thickness of glass wool 50mm, and to be continued right up to the slab level, as per manufacturer's specifications etc. complete. Measurement shall be running length multiplied by height upto true ceiling. |  |  |  |  |  | Sqm. |  |  |  | 382 |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  | N.S. |  |  |  | Full Glazed Partition :- Supply and installation of Full height Glass Partition with 12 mm. thick Toughened Glass resting over Al channel of approved make all complete as per direction of the Architect & EIC. |  |  |  |  |  | Sqm. |  |  |  | 160 |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  | N.S. |  |  |  | Wall Panelling : - Designing, supplying, fabricating and erecting of panelling on walls made with approved plywood frame, placing @ 600 c/c both horizental and vertical fixed with rawal plug on the walls, or any space that needs panelling work in level and plumb with edge finishes as per drawing and direction of E.I.C. Providing and fixing of approved quality and shade 1mm thick laminate with approved adhesive on 12/19mm thick BWR commercial Ply fixed over primary surface as per drawing and direction of E.I.C. |  |  |  |  |  | Sqm. |  |  |  | 201 |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  | N.S. |  |  |  | Column Capital & Panelling :- Designing, supplying, fabricating and erecting of panelling on columns made with approved plywood frame, placing @ 600 c/c both horizental and vertical fixed with rawal plug on the walls, or any space that needs panelling work in level and plumb with edge finishes as per drawing and direction of E.I.C. Providing and fixing of approved quality and shade 1mm thick laminate with approved adhesive on 12 thick BWR commercial Ply fixed over primary surface as per drawing and direction of E.I.C. |  |  |  |  |  | Sqm. |  |  |  | 320.00 |  |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  | 9.73A |  |  |  | Pelmets: - Providing and fixing 18 mm thick. 150mm wide pelmet of flat pressed 3 layer and graded particle board (medium density grade I,con forming to IS 3087) including top cover of 6 mm commercial ply wood, nickel plated M.S.pipe 20 mm dia (heavy type) curtain rod with nickel plated brackets including fixing with 25 x3 mm M.S.Hat 10cm ling and rawl plugs 50mm long ( designation 10 no.)etc. all complete Details of cost for a pelmet 2m long |  |  |  |  |  | Rmt |  |  |  | 160.00 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 5 |  |  |  | WALL PREPARATION & FINISHES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  | 13.80 A.2 |  |  |  | Providing and applying white cement based putty of average thickness 2 mm, of approved brand and manufacturer, over the plastered wall surface to prepare the surface even and smooth complete. |  |  |  |  |  |  |  |  |  | 2464 |  |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  | DSR 13.83.2 |  |  |  | Wall painting with premium acrylic emulsion paint of interior grade, having VOC (Volatile Organic Compound ) content less than 50 grams/ litre of approved and and manufacture, including applying additional coats wherever required to achieve even shade and color. Two coats |  |  |  |  |  | Sqm. |  |  |  | 2464 |  |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  | 13.93.2 |  |  |  | Painting with synthetic enamel paint of approved brand and manufacture of required color to give an even shade : |  |  |  |  |  | Sqm. |  |  |  | 150 |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  | N.S. |  |  |  | Wall Paper: - Providing and fixing of wall paper with surface preparation. Wall paper to be 100% washable. |  |  |  |  |  | Sqm. |  |  |  | 224 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | BUILT-IN JOINERIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  | 9.1 |  |  |  | Providing Wood work in frames of doors, windows, clerestory windows and other frames, wrought framed and fixed in position. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.1.1 |  |  |  | second class teakwood |  |  |  |  |  | Cum |  |  |  | 2.00 |  |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  | 9.20 |  |  |  | Flush Door :- Providing and fixing ISI marked flush door shutters conforming to IS : 2202 (Part I) decorative type, core of block board construction with frame of 1st class hard wood and well matched teak 3 ply veneering with vertical grains or cross bands and face veneers on both faces of shutters.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.20.1 |  |  |  | 35 mm thick including stainless steel butt hinges with necessary screws sqm. |  |  |  |  |  | Smt |  |  |  | 62 |  |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  | 21.18 |  |  |  | Providing and fixing 12 mm thick frameless toughened glass door shutter of approved brand and manufacture, including providing and fixing top & bottom pivot & spring type fixing arrangement and making necessary holes etc. for fixing required door fittings, all complete as per direction of Engineer-in-charge (Door handle, lock and stopper etc.to be paid separately). |  |  |  |  |  | sqm |  |  |  | 88 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Glass Door/window |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  | N.S. |  |  |  | Providing and Fixing Senor operated 12 mm thick Toughened glass door shutter of approved brand and manufacture, Door handle, lock and stopper etc. All complete as per direction of Engineer-in-charge. |  |  |  |  |  | Each |  |  |  | 3 |  |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  | 9.100.1 |  |  |  | Providing and fixing aluminum handles anodized (anodic coating not less than grade AC 10 as per IS:1868) transparent or dyed to required color or shade with necessary screws etc. complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.100.1 |  |  |  | 125 mm |  |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.100.2 |  |  |  | 100 mm |  |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  | 9.96 |  |  |  | Providing and fixing aluminum sliding door bolts anodized (anodic coating not less than grade AC 10 as per IS:1868) transparent or dyed to required color or shade with nuts and screws etc. complete: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.96.2 |  |  |  | 250 x 16 mm |  |  |  |  |  | Each |  |  |  | 30.00 |  |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  | 9.97 |  |  |  | Providing and fixing aluminum tower bolts anodized (anodic coating not less than grade AC 10 as per IS:1868) transparent or dyed to required color or shade with necessary screws etc. complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.97.4 |  |  |  | 150 x 10 mm |  |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.97.5 |  |  |  | 100x10 mm |  |  |  |  |  | Each |  |  |  | 10.00 |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  | 9.98 |  |  |  | Providing and fixing aluminum pull bolt lock anodized (anodic coating not less than grade AC 10 as per IS:1868) transparent or dyed to required color or shade with necessary screws etc. complete. |  |  |  |  |  | Each |  |  |  | 15.00 |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  | 9.88 |  |  |  | Providing and fixing bright finished brass 100 mm mortice latch and lock with six levers and a pair of anodized( anodic coating not less than grade AC 10 as per IS: 1868) aluminum lever with necessary screws etc. Completed Best make of approved Quality). |  |  |  |  |  | Each |  |  |  | 14.00 |  |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  |  | 9.101 |  |  |  | Providing and fixing aluminum hanging floor door stopper anodized (anodic coating not less than grade AC 10 as per IS:1868) transparent or dyed to required color and shade with necessary screws etc. complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9.101.2 |  |  |  | Twin rubber stopper |  |  |  |  |  | Each |  |  |  | 14.00 |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  | 21.4.2 |  |  |  | Providing and fixing double action hydraulic floor spring of approved brand and manufacture conforming to IS : 6315, having brand logo embossed on the body / plate with double spring mechanism and door weight upto 125 kg, for doors, including cost of cutting floors, embedding in floors as required and making good the same matching to the existing floor finishing and cover plates with brass pivot and single piece M.S. sheet outer box with slide plate etc. complete asper the direction of Engineer-in-charge. minimum 1.25 mm Thickness Floor Spring |  |  |  |  |  | Each |  |  |  | 30.00 |  |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  | N.S. |  |  |  | Janitor Closet: - Providing and fixing of janitor closet, depth 450mm with 19mm thick commercial plywood finished in laminate of approved shade. Shelves shall be of removal type and all internal surfaces shall be finished with backing laminate, complete as approved by EIC. |  |  |  |  |  | Sqm. |  |  |  | 15.00 |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  | N.S. |  |  |  | Display Cabinet : - Providing and fixing of Display Cabinet, depth 450mm with 19mm thick commercial plywood finished in laminate of approved shade with openable glass (10mm toughened) and all internal surfaces shall be finished with backing laminate, complete as approved by EIC. |  |  |  |  |  | Sqm. |  |  |  | 25.00 |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  | N.S. |  |  |  | Pantry Counter : -Providing and fixing 600mm wide pantry counter with top in approved sample pre-polished granite slab 18mm thick laid on 25mm thick one side polished cuddapah in cement mortar 1:4 in between, supported on 19mm thick both sides polished cuddapah verticals as directed. Providing 100mm high front facia and band above the counter in same shade granite. All exposed surfaces of platform to be finished in same granite slab. Necessary cut-outs for sink to be provided and all cut-outs, exposed edges to be bull Nosed with mirror polish. Cost to also include making necessary cut-outs for taking Pipes through counter top. Cost to also include of providing 100mm high raised platform in brickbat coba, with polished granite tile skirting as per specifications. All vertical surfaces of wall / partition below counter top to be finished in 200 x 200mm second grade approved make white colour ceramic tiles. Cost to also include making necessary cut-outs for taking drainage pipes and Nahni traps in raised platform, all completed and finished to satisfactory conditions. Cost to be inclusive of shutters of 19mm thick commercial grade plywood finished in approved shade laminate on exposed surface and balancing laminate on internal surfaces inclusive of necessary teak wood beading and frame work finished in colour polish to match the shade of the laminate or as directed. Cost to be inclusive of horizontal shelves in 19mm thick commercial grade plywood finished in approved shade laminate. Hardware to include dead locks with SS finish keyhole, brush SS finish handle, SS tower |  |  |  |  |  | Rmt. |  |  |  | 3.00 |  |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  | N.S. |  |  |  | Laminated A-V Cabinet: - Fabricating and fixing of AV cabinet formed of 19mm ply formwork/carcass with adjustable shelves of laminated 19mm BWR ply. Shutters of cabinet to be formed of 19mm thick ply finished with laminate of approved shade with approved SS handles as per details. Rear backing of cabinet to be formed with 12mm thick ply with laminate finish. Top to be made of 19mm BWR ply finished with laminate of approved shade as per details. All internal surface to be finished in backing laminate. |  |  |  |  |  | Sqm. |  |  |  | 4.00 |  |  |  |  |  |  |  |  |
| 55 |  |  |  |  |  |  | N.S. |  |  |  | Over Head Storage : -Providing and fixing overhead storage of depth 400mm and size as per drawings, with top, sides and bottom in 19mm thick commercial grade plywood, back in 8mm thick commercial grade plywood, intermediate in 19mm thick commercial grade plywood, shutter in 19mm thick commercial grade block board. All external surfaces to be finished in approved laminate, with balancing laminate on the interior surface. All exposed edges to be finished in PVC edge banding. Rate to include hardware such as approved locks, auto-closing hinges for shutters, approved handles, door stop, tower bolt, etc. complete. |  |  |  |  |  | Sqm. |  |  |  | 12.00 |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  | N.S. |  |  |  | Reception Back-drop: - Providing and fabricating reception backdrop, as per approved design and size made of 12/19mm thick commercial plywood finished on all exposed surfaces with 1mm thick designer laminate of approved shade. Surface to house the LOGO in SS matt finish of grade SS 304, all complete as directed by the Architect. |  |  |  |  |  | Nos. |  |  |  | 1.00 |  |  |  |  |  |  |  |  |
| 57 |  |  |  |  |  |  | N.S. |  |  |  | Full Height Storage (2700mm height ) :- Providing and fixing of Full height storage of 2700mm high, depth 450mm with 19mm thick commercial plywood finished in laminate of approved shade. Shelves shall be of removal type and all internal surfaces shall be finished with backing laminate, complete as approved by EIC. |  |  |  |  |  | sqm |  |  |  | 110 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 9 |  |  |  | MISCELLANEOUS ITEMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 58 |  |  |  |  |  |  | N.S. |  |  |  | Frosted Glass Film : - Providing and fixing of frosted sparkle film on glazed partition and door as per approved design by the Architect |  |  |  |  |  | Sqm. |  |  |  | 247 |  |  |  |  |  |  |  |  |
| 59 |  |  |  |  |  |  | N.S. |  |  |  | Vertical Blinds: - Providing and fixing of vertical blinds of approved shade by the Architect, all complete with channel, chord, string, clutch and accessories as required. |  |  |  |  |  | Sqm. |  |  |  | 25.00 |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  | N.S. |  |  |  | Roller Blinds: - Providing and fixing of roller blinds of approved shade by the Architect, all complete with roller, chord, string, clutch and accessories as required. Openness factor of fabric not to exceed 5%. |  |  |  |  |  | Sqm. |  |  |  | 187 |  |  |  |  |  |  |  |  |
| 61 |  |  |  |  |  |  |  |  |  |  | Carriage of materials including transportations , loading, unloading, stacking etc. all complete |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | i) sand ( 45 km lead ) |  |  |  |  |  | Cum |  |  |  | 27 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | iii stone chips ( 11.5 km lead) |  |  |  |  |  | Cum |  |  |  | 2 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | iii) Bricks ( 1 km Kachha +7km pacca ) |  |  |  |  |  | 1000 Nos. |  |  |  | 5600 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Extra |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | i) Cement |  |  |  |  |  | bag |  |  |  | 113 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ii) Bricks |  |  |  |  |  | 1000 Nos. |  |  |  | 5600 |  |  |  |  |  |  |  |  |
| B ) :- PHE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S.N. |  |  |  | S.O.R 2016 |  |  |  |  |  |  | DESCRIPTION |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |  |  |
| I- B. WATER SUPPLY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | 18.50.1 |  |  |  |  |  |  | Providing and fixing C.P. brass long nose bib cock of approved quality conforming to IS standards and weighing not less than 810 gms. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| 2 |  |  |  | 18.51 |  |  |  |  |  |  | Providing and fixing C.P. brass long body bib cock of approved quality conforming to IS standards and weighing not less than 690 gms. 15 mm nominal bore |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| 3 |  |  |  | 18.52 |  |  |  |  |  |  | Providing and fixing C.P. brass stop cock ( concealed) of standard design and of approved make confirming to IS:8931 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 15 mm nominal bore |  |  |  |  | Each |  |  |  | 36.00 |  |  |  |  |  |  |  |
| 4 |  |  |  | 18.21 |  |  |  |  |  |  | Providing and fiixng PVC connection pipe with brass unions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.21.2 |  |  |  |  |  |  | 45 cm length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.21.2.1 |  |  |  |  |  |  | 15 mm nominal bore |  |  |  |  | Each |  |  |  | 36.00 |  |  |  |  |  |  |  |
| 5 |  |  |  | 18.53 |  |  |  |  |  |  | Providing and fixing C.P. brass angle valve for basin mixer and geyser points of approved quality conforming to IS:8931 a) 15 mm nominal bore |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.53.1 |  |  |  |  |  |  | 15mm nominal bore |  |  |  |  | Each |  |  |  | 36.00 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  | Supplying fitting and fixing the following items complete with all materials as per specifications and direction of E/I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a |  |  |  | 17.33 |  |  |  |  |  |  | Providing and fixing 600x120x5mm glass shelf with edges round of supported on anodized aluminum angle frame with C.P brass brackets and guard rail complete fixed with 40mm long screws, rawl plugs etc., complete |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| b |  |  |  | 17.31 |  |  |  |  |  |  | Providing and fixing 600 x 450 mm beveled edge mirror of superior glass ( of approved quality) complete with 6 mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| c |  |  |  | 17.73 |  |  |  |  |  |  | Providing and fixing PTMT towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fittings arrangement of approved quality and color. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.73.2 |  |  |  |  |  |  | 600 mm long towel rail with total length of 645 mm, width 78 mm and effective height of 88 mm, weighing not less than 190 gms. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| d |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing C.P.Matt Chrome finish Toilet Paper Holder, Jaquar make Cat. No.ACN- 1153N, inclusive of taxes royalty all complete as per specification and direction of Engineer-in-charge. |  |  |  |  | Each |  |  |  | 8.00 |  |  |  |  |  |  |  |
| E. |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing Towel ring Jaquar make Cat. No.ACN1121N inclusive of taxes royalty all complete as per specification and direction of Engineer-in-charge. |  |  |  |  | Each |  |  |  | 7.00 |  |  |  |  |  |  |  |
| F. |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing Soap Dish , Jaquar make ACN.1135 N , inclusive of taxes royalty all complete as per specification and direction of Engineer-in-charge. |  |  |  |  | Each |  |  |  | 7.00 |  |  |  |  |  |  |  |
| 7 |  |  |  | 18.1 |  |  |  |  |  |  | Providing and fixing G.I pipes complete with G.I fittings and clamps, including and making good the wall etc. (internal work) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Exposed on wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.10.2 |  |  |  |  |  |  | 20 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 42.00 |  |  |  |  |  |  |  |
|  |  |  |  | 18.10.3 |  |  |  |  |  |  | 25 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 94.00 |  |  |  |  |  |  |  |
| 8 |  |  |  | 18.11 |  |  |  |  |  |  | Providing and fixing G.I. concealed pipe including painting with anti-corrosive bitumastic paint, cutting chases and making good the wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.11.1 |  |  |  |  |  |  | 15 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 102.00 |  |  |  |  |  |  |  |
|  |  |  |  | 18.11.2 |  |  |  |  |  |  | 20 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 84.00 |  |  |  |  |  |  |  |
| 9 |  |  |  | 18.38. |  |  |  |  |  |  | Painting G.I. pipes and fittings with synthetic enamel white paint with two coats over a ready mixed priming coat, both of approved quality for new work : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.38.2 |  |  |  |  |  |  | 20 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 42.00 |  |  |  |  |  |  |  |
|  |  |  |  | 18.38.3 |  |  |  |  |  |  | 25 mm dia nominal bore |  |  |  |  | Metre |  |  |  | 94.00 |  |  |  |  |  |  |  |
| 10 |  |  |  | 18.17 |  |  |  |  |  |  | Providing and fixing gun metal gate valve with C.I wheel of approved quality (screwed end) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 18.17.2 |  |  |  |  |  |  | 32mm nominal bore |  |  |  |  | Each |  |  |  | 7.00 |  |  |  |  |  |  |  |
|  |  |  |  | 18.17.3 |  |  |  |  |  |  | 40 mm nominal bore |  |  |  |  | Each |  |  |  | 3.00 |  |  |  |  |  |  |  |
| 11 |  |  |  | 17.28 |  |  |  |  |  |  | Providing and fixing P.V.C waste pipe for sink or wash basin including P.V.C waste fittings complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.28.1 |  |  |  |  |  |  | semi rigid pipe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.28.1.1 |  |  |  |  |  |  | 32mm dia |  |  |  |  | Each |  |  |  | 25.00 |  |  |  |  |  |  |  |
| II- SANITARY INSTALLATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | 17.1 |  |  |  |  |  |  | Providing and fixing water closet squatting pan (Indian type W.C pan) with 100 mm P or S -trap 10 litre low level white P.V.C flushing cistern with manually controlled device (handle lever) conforming to IS:7231, with all fittings and fixtures complete including cutting and making good the walls and floors wherever required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.1.1 |  |  |  |  |  |  | White Vitreous china Orissa patterns W.C pan of size 580 mm x 440 mm with integral type foot rest. |  |  |  |  | Each |  |  |  | 8.00 |  |  |  |  |  |  |  |
| 13 |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing white color vitreous china wall mounted European type water closet Parryware VERVE C0295 mounting Constellation with seat cover and accessories, Pipe work Concealed behind wall & Cast iron Chair Bracket and Rag Bolts flush bend, overflow arrangement with specials of standard make and mosquito proof coupling of approved municipal design complete including painting of fittings and brackets , cutting and making good the walls and floors wherever required: ( Hindware /Shera / Parryware & equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | W.C. pan with ISI marked white solid plastic seat |  |  |  |  | Each |  |  |  | 4.00 |  |  |  |  |  |  |  |
| 14 |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing white colored counter Top Wash Basin PARRYWARE C848H all complete inclusive of taxes and royalty as per specification and direction of Engineer-in-charge. ( Hindware /Shera / Parryware & equivalent make ) |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| 15 |  |  |  | M.R. |  |  |  |  |  |  | Providing and fixing white color vitreous china URINAL with flush fittings Parryware make Whiz C0580 & C8123 including painting of fittings and brackets , cutting and making good the walls and floors wherever required: ( Hindware /Shera / Parryware & equivalent make ) |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |  |
| 16 |  |  |  | 17.10 |  |  |  |  |  |  | Providing and fixing Stainless Steel A ISI 304 (18/8) kitchen sink as per IS : 13983 with C.I. brackets and stainless steel plug 40 mm, including painting of fittings and brackets, cutting and making good the walls wherever required : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.10.1.3 |  |  |  |  |  |  | 510x1040 mm bowl depth 200 mm |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |  |
| 17 |  |  |  | 12.78.2 |  |  |  |  |  |  | Providing and fixing on wall face unplasticised Rigid PVC rain water pipes conforming to IS : 13592 Type A, including jointing with seal ring conforming to IS : 5382, leaving 110 mm gap for thermal expansion, (i) Single socketed pipes. |  |  |  |  | Metre |  |  |  | 237.00 |  |  |  |  |  |  |  |
| 18 |  |  |  | 12.79 |  |  |  |  |  |  | Providing and fixing on wall face unplasticised -PVC moulded fittings/accessories for unplasticised -Riging PVC pipes conforming to IS:4985 including jointing with seal ring conforming to IS: 5382 leaving 10mm gap for thermal expansion. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  |  |  | 12.79.3.2 |  |  |  |  |  |  | Single tee with door |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 110 x 110x 110 mm |  |  |  |  | Each |  |  |  | 10.00 |  |  |  |  |  |  |  |
| b |  |  |  | 12.79.4 |  |  |  |  |  |  | single tee without door |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12.79.4.2 |  |  |  |  |  |  | 110 x 110x 110 mm |  |  |  |  | Each |  |  |  | 24.00 |  |  |  |  |  |  |  |
| C |  |  |  | 12.79.5.2 |  |  |  |  |  |  | Bend |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 12.79.5.2 |  |  |  |  |  |  | 110 mm bend |  |  |  |  | Each |  |  |  | 72.00 |  |  |  |  |  |  |  |
| D |  |  |  | 12.80 |  |  |  |  |  |  | Providing and fixing unplasticised -PVC pipe clips of approved design to unplasticised-PVC rain water pipes by means of 50 x 50 x 50mm hard wood plugs, screwed with M.s screws of required length including cutting brick work and fixing in cement mortar 1;4 (1 cement: 4 coarse sand) and making good the wall etc. complete.110 mm |  |  |  |  | Each |  |  |  | 60.00 |  |  |  |  |  |  |  |
| 19 |  |  |  | 17.60 |  |  |  |  |  |  | Providing and fixing trap of self-cleansing design with screwed down or hinged grating with or without vent arm complete, including cost of cutting and making good the walls and floors. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 17.60.1 |  |  |  |  |  |  | 100 mm inlet and 100 mm outlet |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | III - SEWERAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | 19.6 |  |  |  |  |  |  | Providing and laying non-pressure NP2 class (light duty) R.C.C pipes with collars jointed with stiff mixture of cement mortar in the proportion of 1:2 (1 cement: 2 fine sand) including testing of joints etc. complete. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.6.2 |  |  |  |  |  |  | 150 mm dia R.C.C pipe |  |  |  |  | Metre |  |  |  | 60.00 |  |  |  |  |  |  |  |
| 21 |  |  |  | 2.10.1.2 |  |  |  |  |  |  | Excavating trenches of required width for pipes, cables, etc including excavation for sockets, and dressing of sides, ramming of bottoms, depth upto 1.5 m including getting out the excavated soil, and then returning the soil as required, in layers not exceeding 20 cm in depth including consolidating each deposited layer by ramming, watering, etc. and disposing of surplus excavated soil as directed, within a lead of 50 m. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | All kinds of soil. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Pipes, cables etc. exceeding 80 mm dia. but not exceeding 300 mm dia. |  |  |  |  | Metre |  |  |  | 60.00 |  |  |  |  |  |  |  |
| 22 (a) |  |  |  | 19.7 |  |  |  |  |  |  | Constructing brick masonry manhole with 75 class designation bricks in cement mortar 1:4 (1 cement: 4 coarse sand) R.C.C top slab with 1:2:4 mix (1 cement: 2 coarse sand: 4 graded stone aggregate 20mm nominal size), foundation concrete 1:4:8 mix ( 1 cement: 4 coarse sand: 8 graded stone aggregate 40mm nominal size) inside plastering 12mm thick with cement mortar 1:3 ( 1cement: 3 coarse sand) finished with floating coat of neat cement and making channels in cement concrete 1:2:4 ( 1 cement: 2 coarsesand:4 graded stone aggregate 20mm nominal size) finished with a floating coat of neat cement complete as per standard design: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.7.1 |  |  |  |  |  |  | Inside size 90x80 cm and 45 cm deep including C.I cover with frame (light duty) 455 x 610mm internal dimensions, total weight of cover and frame to be not less than 38 kg (weight of cover 23 kg and weight of frame of 15 kg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.7.1.1 |  |  |  |  |  |  | With F.P.S bricks |  |  |  |  | Each |  |  |  | 8.00 |  |  |  |  |  |  |  |
| (b) |  |  |  | 19.7.2 |  |  |  |  |  |  | Inside size 120 x 90cm deep including C.I cover with frame (medium duty) 500mm internal diameter, total weight of cover and frame to be not less than 116 kg (weight of cover 58 kg and weight of frame 58 kg.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.7.2.1 |  |  |  |  |  |  | With F.P.S Bricks |  |  |  |  | Each |  |  |  | 5.00 |  |  |  |  |  |  |  |
| 23 |  |  |  | 19.4 |  |  |  |  |  |  | Providing and fixing square-mouth S.W gully trap grade 'A' complete with C.I grating brick masonry chamber with water tight C.I cover with frame of 300 x 300mm size (inside) the weight of cover to be not less than 4.50 kg and frame to be not less than 2.70 kg as per standard design. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.4.3 |  |  |  |  |  |  | 180 x 150mm size P type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 19.4.3.1 |  |  |  |  |  |  | With F.P.S Bricks class designation 75 |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |  |
| Item No. |  |  |  | Ref |  |  |  |  |  |  | Description |  |  |  |  | Unit |  |  |  | Quantity |  |  |  |  |  |  |  |
| C : ELECTRICAL WORKS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1.01 |  | 1.3.3 BSR 2016 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Group C |  |  |  |  | Point |  |  |  | 485.00 |  |  |  |  |  |  |
| 3.1.02 |  | 1. 4.BSR |  |  |  |  |  |  |  | : Wiring for twin control light point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface / recessed PVC conduit, 2 way modular switch, modular plate, suitable G.I. box etc. as required. |  |  |  |  | Point |  |  |  | 2.00 |  |  |  |  |  |  |
| 3.1.03 |  | 1.5 BSR |  |  |  |  |  |  |  | : Wiring for light / power plug with 2 x 4 sq. mm FR PVC insulated copper conductor single core cable in surface / recessed PVC conduit alongwith 1 no. 4 sq. mm FR PVC insulated copper conductor single core cable for loop earthing as required. |  |  |  |  | Meter |  |  |  | 1500.00 |  |  |  |  |  |  |
| 3.1.04 |  | 1.7. BSR |  |  |  |  |  |  |  |
| (a) |  | 1.7.1 |  |  |  |  |  |  |  | 2 x 1.5 sq.mm. + 1 x 1.5 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 650.00 |  |  |  |  |  |  |
| (b) |  | 1.7.2 |  |  |  |  |  |  |  | 2 x 2.5 sq.mm. + 1 x 2.5 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 1650.00 |  |  |  |  |  |  |
| (c ) |  | 1.7.4 |  |  |  |  |  |  |  | 2 x 6 sq.mm. + 1 x 6 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 140.00 |  |  |  |  |  |  |
|  |  | 1.7.11 |  |  |  |  |  |  |  | 4 x 6 sq.mm. + 1 x 6sq. mm earth wire |  |  |  |  | Meter |  |  |  | 150.00 |  |  |  |  |  |  |
| (d) |  | 1.7.12 |  |  |  |  |  |  |  | 4 x 10 sq.mm. + 1 x 10 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 160.00 |  |  |  |  |  |  |
| (e) |  | 1.7.13 |  |  |  |  |  |  |  | 4 x 16 sq.mm. + 1x 16 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 160.00 |  |  |  |  |  |  |
| (f) |  | 1.7.14BSR |  |  |  |  |  |  |  | 4 x 25 sq.mm. + 1 x 16 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 15.00 |  |  |  |  |  |  |
| 3.1.05 |  | 1.16. BSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 2 Pair |  |  |  |  | Metre |  |  |  | 550.00 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 4Pair |  |  |  |  | Metre |  |  |  | 500.00 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 10 Pair |  |  |  |  | Metre |  |  |  | 450.00 |  |  |  |  |  |  |
|  |  | MR |  |  |  |  |  |  |  | : Supplying , fixing, fitting and connection of 50 pair telephone D.P. BOX, all complete as required |  |  |  |  | Each |  |  |  | 3.00 |  |  |  |  |  |  |
|  |  | MR |  |  |  |  |  |  |  | : Supplying , fixing, fitting and connection of 200 pair telephone D.P. BOX, all complete as required |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
| 3.1.06 |  | 1.17 BSR |  |  |  |  |  |  |  | : Supplying and drawing co-axial TV cable RG-6 grade, 0.7 mm solid core copper conductor PE insulated, shielded with fine tinned copper braid and protected with PVC sheath in the existing surface / recessed steel / PVC conduit as required. |  |  |  |  | Metre |  |  |  | 70.00 |  |  |  |  |  |  |
| 3.1.07 |  | 1.21. BSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 20mm |  |  |  |  | Metre |  |  |  | 445.00 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 25 mm |  |  |  |  | Metre |  |  |  | 350.00 |  |  |  |  |  |  |
| 3.1.08 |  | 1.24 BSR |  |  |  |  |  |  |  |
| (a) |  | 1.24.1 |  |  |  |  |  |  |  | 6/10A switch |  |  |  |  | Each |  |  |  | 60.00 |  |  |  |  |  |  |
| (b) |  | 1.24.4 |  |  |  |  |  |  |  | 5 pin 5/6 A socket outlet |  |  |  |  | Each |  |  |  | 60.00 |  |  |  |  |  |  |
| (C) |  | 1.24.6 |  |  |  |  |  |  |  | Telephone socket outlet |  |  |  |  | Each |  |  |  | 150.00 |  |  |  |  |  |  |
| (d) |  | 1.24.5 |  |  |  |  |  |  |  | 6pin 16/20 A socket outlet |  |  |  |  | Each |  |  |  | 0.00 |  |  |  |  |  |  |
| (e) |  | 1.24.3 |  |  |  |  |  |  |  | 16/20A switch |  |  |  |  | Each |  |  |  | 0.00 |  |  |  |  |  |  |
| (f) |  | 1.24.7 |  |  |  |  |  |  |  | TV antena socket outlet |  |  |  |  | Each |  |  |  | 2.00 |  |  |  |  |  |  |
| 3.1.09 |  | 1.27. BSR |  |  |  |  |  |  |  |
|  |  | 1.27.1 |  |  |  |  |  |  |  | 1 or 2 Module ( 78 mm x 78 mmx45mm) |  |  |  |  | Each |  |  |  | 152.00 |  |  |  |  |  |  |
|  |  | 1.27.2 |  |  |  |  |  |  |  | 3 Module ( 100 mm x 78 mmx45mm) |  |  |  |  | Each |  |  |  | 18.00 |  |  |  |  |  |  |
|  |  | 1.27.3 |  |  |  |  |  |  |  | 4 Module ( 140mm x 78 mmx50mm) |  |  |  |  | Each |  |  |  | 18.00 |  |  |  |  |  |  |
|  |  | 1.27.4 |  |  |  |  |  |  |  | 6 Module ( 205 mm x 78mmx50mm) |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |
|  |  | 1.27.5 |  |  |  |  |  |  |  | 8 Module ( 230mmx78 mm x 60mm) |  |  |  |  | Each |  |  |  | 20.00 |  |  |  |  |  |  |
|  |  | 1.27.6 |  |  |  |  |  |  |  | 12Module ( 135 mm x205mmx 60 mm) |  |  |  |  | Each |  |  |  | 16.00 |  |  |  |  |  |  |
| 3.1.11 |  | 1.26 BSR |  |  |  |  |  |  |  | : Supplying and fixing blanking plate on the existing modular plate & switch box excluding modular plate as required. |  |  |  |  | Each |  |  |  | 100.00 |  |  |  |  |  |  |
| 3.1.12 |  | 1.31 BSR |  |  |  |  |  |  |  | : Supplying and fixing suitable size GI box with modular plate and cover in front on surface or in recess including providing and fixing 5 pin 5/6 amps modular socket outlet and 5/6 amps modular switch, connection, painting etc. as required.( For light plugs to be used in nonresidential buildings.) |  |  |  |  | Each |  |  |  | 25.00 |  |  |  |  |  |  |
| 3.1.13 |  | 1.32 BSR |  |  |  |  |  |  |  | : Supplying and fixing suitable size GI box with modular plate and cover in front on surface or in recess, including providing and fixing 6 pin 20/16 amps modular socket outlet and 20/16 amps modular switch, connection, painting etc. as required. |  |  |  |  | Each |  |  |  | 60.00 |  |  |  |  |  |  |
| 3.1.14 |  | 2.19 BSR |  |  |  |  |  |  |  | :Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 20 amps 'C' series, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required. |  |  |  |  | Each |  |  |  | 55.00 |  |  |  |  |  |  |
| 3.1.15 |  | 1.33 BSR |  |  |  |  |  |  |  | : Supplying and fixing 2 pin, 5 amp. Ceiling rose on the existing junction box / wooden block including connection etc. as required. |  |  |  |  | Each |  |  |  | 25.00 |  |  |  |  |  |  |
| 3.1.16 |  | 1.25 BSR |  |  |  |  |  |  |  | : Supplying and fixing stepped type electronic fan regulator on existing modular plate switch box including connection etc. as required. |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
| 3.1.17 |  | 1.34 BSR |  |  |  |  |  |  |  | : Supplying and fixing batten / angle holder including connection etc. as required. |  |  |  |  | Each |  |  |  | 18.00 |  |  |  |  |  |  |
| 3.2.01 |  | 5.11..4 |  |  |  |  |  |  |  | Supplying and fixing recess mounted 12W LED vertical down lighter luminaire complete with all accessories and replaceable electronic driver, directoy on false ceiling including connection with 1.5 sq.mm.FR PVC INSULATED copper conductor cable and earthing the body as required. |  |  |  |  | Each |  |  |  | 180.00 |  |  |  |  |  |  |
| 3.2.02 |  | 5.11.3 |  |  |  |  |  |  |  | Supplying and fixing recess mounted 6/7 W LED vertical down lighter luminaire complete with all accessories and replaceable electronic driver, directoy on false ceiling including connection with 1.5 sq.mm.FR PVC INSULATED copper conductor cable and earthing the body as required. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |
| 3.2.05 |  | 5.10.3. |  |  |  |  |  |  |  | Supplying and fixing indoor recess mounted 36 W LED flat panel luminaire suitable for armstrong /grid ceiling ,complete with electronic driver etc. directory on false ceiling including connection with 1.5 sq.mm.FR PVC INSULATED copper conductor cable and earthing the body as required. |  |  |  |  | Each |  |  |  | 96.00 |  |  |  |  |  |  |
| 3.2.06 |  | BSR 1.37 |  |  |  |  |  |  |  | Providing and fixing Ding Dong Bell for surface Mounting all-inclusive as per specification and direction of E/I. |  |  |  |  | Each |  |  |  | 10.00 |  |  |  |  |  |  |
| 3.2.07 |  | 8.11.7 |  |  |  |  |  |  |  | Supply and installation of 400 mm 230 v AC wall /cabin fan including fixing the bracket , making connection, testing , commissioning etc. as required. |  |  |  |  | Each |  |  |  | 120.00 |  |  |  |  |  |  |
| 3.2.08 |  | 8.11.4 |  |  |  |  |  |  |  | Supply and installation of 300 mm 230 v AC exhaust fan in the existing opening including making good the damage , connection , testing, commissioning etc. as required. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |
|  |  | 5.9.2 |  |  |  |  |  |  |  | Providing and fixing following powder coated batten for 28 watt T5 fluorescent tube, complete with all accessories directly on ceiling/wall including connection with 1.5 sq.mm FR pvc insulated copper conductor cable etc. all complete as required. |  |  |  |  | Each |  |  |  | 32.00 |  |  |  |  |  |  |
| 3.2.09 |  | 5.3.1 |  |  |  |  |  |  |  | Providing and fixing following mounting rail /box type fluorescent fitting of all type complete with all accessories, 1x4’-40w fluorescent lamp, etc. directly on ceiling/wall including connection with 1.5 sq.mm FR pvc insulated copper conductor cable etc. All complete as required. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |
| 3.2.10 |  | 6.1.7 |  |  |  |  |  |  |  | Providing and fixing following types of lamp in the existing electrical fitting/fixture, bracket holder etc. all complete as required. |  |  |  |  | Each |  |  |  | 12.00 |  |  |  |  |  |  |
| 3.3.01 |  | 2.9.4 BSR |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 4 way (4 + 12), Double door, horizontal type |  |  |  |  | Each |  |  |  | 9.00 |  |  |  |  |  |  |
|  |  | 2.8.8 |  |  |  |  |  |  |  | 8 way Double door SPN DB |  |  |  |  | Each |  |  |  | 3.00 |  |  |  |  |  |  |
|  |  | 2.8.9 |  |  |  |  |  |  |  | 12 way Double door SPN DB |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
|  |  | 2.9.5 |  |  |  |  |  |  |  | 6 way (4 + 18), Double door, horizontal TPN DB. |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
| 3.3.02 |  | BSR 2.12.1 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Single pole |  |  |  |  | Each |  |  |  | 312.00 |  |  |  |  |  |  |
| 3.3.03 |  | BSR 2.17. |  |  |  |  |  |  |  |
|  |  | 2.17.02 |  |  |  |  |  |  |  | 40 amps |  |  |  |  | Each |  |  |  | 3.00 |  |  |  |  |  |  |
|  |  | 2.17.03 |  |  |  |  |  |  |  | 63 amps |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
|  |  | 2.12.7 |  |  |  |  |  |  |  |
|  |  | 2.12.6 |  |  |  |  |  |  |  | 40 amps |  |  |  |  | Each |  |  |  | 9.00 |  |  |  |  |  |  |
|  |  | 2.12.7 |  |  |  |  |  |  |  | 63 amps |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
| 3.4.01 |  | BSR 7.1.4,2215+ C.P. @15% |  |  |  |  |  |  |  | Supplying and laying of XLPE /PVC insulated PVC sheathed (Heavy Duty) Armoured Electric Cables with aluminum conductor for working voltage 1100 volts to IS:7098 (Part-1) / IS:1554 (Part-1) as applicable, All complete as per building specification and direction of E/I. 3.5 core 400 sqm |  |  |  |  | Meter |  |  |  | 95.00 |  |  |  |  |  |  |
| 3.4.01 |  | BSR 7.1.4,2214+ C.P. @15% |  |  |  |  |  |  |  | Supplying and laying of XLPE /PVC insulated PVC sheathed (Heavy Duty) Armoured Electric Cables with aluminum conductor for working voltage 1100 volts to IS:7098 (Part-1) / IS:1554 (Part-1) as applicable, All complete as per building specification and direction of E/I. 3.5 core 300 sqm |  |  |  |  | Meter |  |  |  | 95.00 |  |  |  |  |  |  |
|  |  | BSR 7.1.2.,2209+ C.P. @15% |  |  |  |  |  |  |  | Supplying and laying of XLPE /PVC insulated PVC sheathed (Heavy Duty) Armoured Electric Cables with aluminum conductor for working voltage 1100 volts to IS:7098 (Part-1) / IS:1554 (Part-1) as applicable, All complete as per building specification and direction of E/I. 3.5 core 95 sqm |  |  |  |  | Meter |  |  |  | 125.00 |  |  |  |  |  |  |
| 3.4.01 |  | BSR 7.1.2.,2207+ C.P. @15% |  |  |  |  |  |  |  | Supplying and laying of XLPE /PVC insulated PVC sheathed (Heavy Duty) Armoured Electric Cables with aluminum conductor for working voltage 1100 volts to IS:7098 (Part-1) / IS:1554 (Part-1) as applicable, All complete as per building specification and direction of E/I. 3.5 core 50 sqm |  |  |  |  | Meter |  |  |  | 75.00 |  |  |  |  |  |  |
| 3.4.02 |  | BSR 2.7.9 |  |  |  |  |  |  |  | Providing and fixing 4 pole ON-load/panel mounted Changeover Switch ,IN EXISTING PANEL BOARD ,400 Amp Rating, making connection and earthing the body etc. All complete as per specification and direction of E/I. |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
| 3.4.03 |  | 8.10.4 |  |  |  |  |  |  |  | Providing and fixing digital volt meter 3 phase in existing cubicle panel board including drilling holes in panel board, making connection etc. All complete as required. |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
| 3.4.04 |  | RA 4 |  |  |  |  |  |  |  | Providing and fixing Indicator Lamp LED type all complete as per specification and direction of E/I. |  |  |  |  | Each |  |  |  | 18.00 |  |  |  |  |  |  |
| 3.4.05 |  | 8.10.6 |  |  |  |  |  |  |  | Providing and fixing digital ammeter 3 phase in existing cubicle panel board including drilling holes in panel board, making connection etc. All complete as required. |  |  |  |  | Meter |  |  |  | 6.00 |  |  |  |  |  |  |
| 3.4.06 |  | BSR 8.10.2. |  |  |  |  |  |  |  | Supplying and fixing wall / floor mounted cubicle panel board fabricated with 16 SWG MS sheet, suitable for panel mounted switch disconnector, including busbar chamber, insulator and Painting etc .as required. (but without busbar copper strip) |  |  |  |  | sft. |  |  |  | 90.00 |  |  |  |  |  |  |
| 3.4.7 |  | BSR 2.3.9 |  |  |  |  |  |  |  | Providing and fixing 400 A / 415 V cubical Switch Fuse unit inside the existing cubicle panel board with ISI marked HRC fuses including drilling holes in cubical panel ,making connection etc.as required. |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
|  |  | BSR 2.4.2 |  |  |  |  |  |  |  | S/F MCCB in cubicle panel board: Providing and fixing following rating and breaking capacity MCCB in existing cubicle panel board including drilling holes in cubicle panel, making connections, etc. as required.160 Amp, 35KA |  |  |  |  | Each |  |  |  | 9.00 |  |  |  |  |  |  |
| 3.4.8 |  | BSR 2.4.2 |  |  |  |  |  |  |  | S/F MCCB in cubicle panel board: Providing and fixing following rating and breaking capacity MCCB in existing cubicle panel board including drilling holes in cubicle panel, making connections, etc. as required.25/100 Amp, 16KA |  |  |  |  | Each |  |  |  | 30.00 |  |  |  |  |  |  |
|  |  | BSR 2.5.6 |  |  |  |  |  |  |  | Supplying and fixing 500 amps. TPN , copper busbar complete with all accessories as required |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
| 3.4.9 |  | BSR 2.5.3 |  |  |  |  |  |  |  | Supplying and fixing 200 amps. TPN , copper busbar complete with all accessories as required |  |  |  |  | Each |  |  |  | 6.00 |  |  |  |  |  |  |
| 3.4.13 |  | BSR 3.13. |  |  |  |  |  |  |  | proving and laying earth connection from earth electrode with 4 mm dia copper wire in 15mm dia G.I. Pipe from earth electrode including connection with, copper thimble, excavation and refilling etc ., a as required. |  |  |  |  | Meter |  |  |  | 50.00 |  |  |  |  |  |  |
| 3.4.14 |  | BSR 3.6. |  |  |  |  |  |  |  | Earthing with copper earth plate 600 mm X 600mmX3mm thick including accessories and providing masonry enclosure with cover plate having locking arrangement and watering pipe of 2.7 meter long etc . With charcoal /cock and salt as required. . |  |  |  |  | each |  |  |  | 4.00 |  |  |  |  |  |  |
| 3.4.15 |  | BSR 3.19 |  |  |  |  |  |  |  | Supplying and fixing 4 mm dia copper wire on surface or, in recess for loop earthing along with existing surface/recessed conduit / submain wiring/cable as required. |  |  |  |  | Meter |  |  |  | 80.00 |  |  |  |  |  |  |
|  |  | ESTIMATE FOR UPS WIRING SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | SUBHEAD 2: UPS WIRING |  |  |  |  | Unit |  |  |  | quantity |  |  |  |  |  |  |
|  |  | 8.10.2+2.4.3+2.5.3+2.15.1+2.15.2+8.10.4 |  |  |  |  |  |  |  | Supplying installation , connection and testing of cubical type ups panel ,made of 16 gauge steel sheet ,dust and vermin proof ,powder coated paint ,compartlised ,lockable type PANEL (3'X2') ,comprising with following :- . 100A TPN MCCB -1NO. OUT GOING 40ASPN 3 NOS., 63A SPN MCB 6 NOS. , 200A BUSBAR, -1 NO., volt meter, indicator lamp etc. all complete as required. |  |  |  |  | Each |  |  |  | 1.00 |  |  |  |  |  |  |
|  |  | BSR 1.27.6+1.24.1+1.24.3, +1.24.4.+1.24.5 |  |  |  |  |  |  |  | : Supplying and fixing suitable size GI box with modular plate and cover in front on surface or in recess including providing and fixing 2 NOS 3 pin 5/6 amps modular socket outlet and 2 NOS 5/6 amps modular switch, and 1 no. 16A sw. & socket for raw power ,connection, etc. as required. (For light plugs to be used in non-residential buildings.) |  |  |  |  | each |  |  |  | 160.00 |  |  |  |  |  |  |
|  |  | 1.7. BSR |  |  |  |  |  |  |  | : Wiring for circuit / submain wiring along with earthwire with the following sizes of PVC insulated copper conductor, single core cable in surface / recessed PVC conduit as required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1.7.1 |  |  |  |  |  |  |  | 2 x 1.5 sq.mm. + 1 x 1.5 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 500.00 |  |  |  |  |  |  |
|  |  | 1.7.2 |  |  |  |  |  |  |  | 2 x 2.5 sq.mm. + 1 x 2.5 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 1600.00 |  |  |  |  |  |  |
|  |  | 1.7.4 |  |  |  |  |  |  |  | 2 x 6 sq.mm. + 1 x 6 sq. mm earth wire |  |  |  |  | Meter |  |  |  | 400.00 |  |  |  |  |  |  |
| SL NO |  |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |
| A |  |  |  |  | DATA & VOICE PASSIVE COMPONENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | Category 6 Cable 305 mtr Roll for data |  |  |  |  |  |  |  |  |  | RMT |  |  |  | 13500 |  |  |  |  |  |
| 2 |  |  |  |  | Category 6 I/O keystone type for data. |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 198 |  |  |  |  |  |
| 3 |  |  |  |  | 1 mtr Patch Cord category 6 for data |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 144 |  |  |  |  |  |
| 4 |  |  |  |  | 2 Mtr Patch cord category 6 for data |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 154 |  |  |  |  |  |
| 5 |  |  |  |  | 24 Port Patch Panel keystone type data |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 9 |  |  |  |  |  |
| 6 |  |  |  |  | Single Face Plate |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 18 |  |  |  |  |  |
| 7 |  |  |  |  | Duplex Face Plate |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 116 |  |  |  |  |  |
| 8 |  |  |  |  | Quard face Plate |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 24 |  |  |  |  |  |
| 9 |  |  |  |  | Category 6 I/O keystone type for voice |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 168 |  |  |  |  |  |
| 10 |  |  |  |  | 24 Port Patch Panel keystone type Voice Field side |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 7 |  |  |  |  |  |
| 11 |  |  |  |  | 24 Port Patch Panel keystone type Voice EPABX side |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 7 |  |  |  |  |  |
| 12 |  |  |  |  | 42U, 800 mm wide x 1000mm deepth Server rack with the following specifications, |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| a |  |  |  |  | 4 fans at top for ventilation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b |  |  |  |  | Front perforated door with lock & key, rear perforated split door with lock & key. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c |  |  |  |  | Castor wheels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d |  |  |  |  | 2 nos Power bar with 14 nos IEC C13 sockets. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| e |  |  |  |  | Lables for Patch Panel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f |  |  |  |  | ESD Port Kit & ESD Wrist Strap. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 1 Mtr Patch Cord category 6 for Voice |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 139 |  |  |  |  |  |
| 14 |  |  |  |  | 100 pair MDF with Module |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 2 |  |  |  |  |  |
| B |  |  |  |  | DATA & VOICE ACTIVE COMPONENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | Wire Less Access Point (Indoor) |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 12 |  |  |  |  |  |
| 2 |  |  |  |  | Power injector for wireless Access Point |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 12 |  |  |  |  |  |
| 3 |  |  |  |  | Wall mount kit for wireless access |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 12 |  |  |  |  |  |
| 4 |  |  |  |  | 24-port switch (10/100/1000 MBPS ,Unmanaged) |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 7 |  |  |  |  |  |
| 5 |  |  |  |  | 2504 Wireless controller with 15 AP License |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 6 |  |  |  |  | Wireless LAN controller with License |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 7 |  |  |  |  | EPABX having active 166 analog ,4 digital extensions with PRI system |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 8 |  |  |  |  | Telephone set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a |  |  |  |  | Analogue set |  |  |  |  |  |  |  |  |  | NOS |  |  |  | RO |  |  |  |  |  |
| b |  |  |  |  | Digital set |  |  |  |  |  |  |  |  |  | NOS |  |  |  | RO |  |  |  |  |  |
| SL NO |  |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |
|  |  |  |  |  | PUBLIC ADDRESS SYSTEM WITH VOICE EVACUATION SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | Supply, installation, testing and commissioning of audio amplifiers Conforming 480 watt RMS. (Bosch ) |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 2 |  |  |  |  | Supplying, installing testing and commissioning approved make 6W wall mounted SPEAKER in stove enameled M.S. sheet steel housing etc., complete. |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 70 |  |  |  |  |  |
| 3 |  |  |  |  | Supplying, laying, testing and commissioning 40 strand twin core control CABLE of PVC insulated copper conductor. |  |  |  |  |  |  |  |  |  | RM |  |  |  | 800 |  |  |  |  |  |
| 4 |  |  |  |  | Supply, Installation, testing and commissioning of 25mm PVC CONDUIT. |  |  |  |  |  |  |  |  |  | RM |  |  |  | 800 |  |  |  |  |  |
| 5 |  |  |  |  | Audio controller for special cabin area only |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 10 |  |  |  |  |  |
| 6 |  |  |  |  | 7 Zone controller with all call controller & table top gooseneck mic. (Standard) |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 7 |  |  |  |  | 16U wall mounted Rack assembly made of aluminum channels side and rear panels made of 16 SWG powder coated steel sheet Front transparent perplex door Bearing with rails, and rollers, 2 fans at the top for ventilation. |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| 8 |  |  |  |  | DVD Player. |  |  |  |  |  |  |  |  |  | NOS |  |  |  | 1 |  |  |  |  |  |
| F :- CC TV WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SL NO |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |  |
|  |  |  | CCTV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Supply and Installation of 1 Megapixel HD dome camera. |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 20 |  |  |  |  |  |  |
| 2 |  |  | Supply and Installation of 2 Megapixel HD dome camera. |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 12 |  |  |  |  |  |  |
| 3 |  |  | Supply and Installation of DVR with 32 channel camera & audio |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |  |
| 4 |  |  | 4 TB hard disk capacity expandable upto 12 TB |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |  |
| 5 |  |  | Power supply for 32 nos camara. |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |  |
| 6 |  |  | 24 inch LED Monitor. |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |  |
| 7 |  |  | Supply and laying of Cat-6 data cables in PVC conduit. |  |  |  |  |  |  |  |  |  |  |  |  |  | Mtr |  |  |  | 1000 |  |  |  |  |  |  |
| 8 |  |  | System commissioning charges with connector etc.. |  |  |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |  |
| SL NO |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |  |
| G - ACCESS CONTROL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | Supply and laying of 6C x 0.75 sq.mm tinned copper conductor, inner sheathed and shielded signal cables in PVC conduit. |  |  |  |  |  |  |  |  |  |  |  | Mtr |  |  |  | 150 |  |  |  |  |  |
| 2 |  |  |  |  | Supply and laying of 2C x 1.0 sq.mm tinned copper conductor, inner sheathed and shielded signal cables in PVC conduit. |  |  |  |  |  |  |  |  |  |  |  | Mtr |  |  |  | 150 |  |  |  |  |  |
| 3 |  |  |  |  | Supply and laying of Cat-6 data cables in PVC conduit. |  |  |  |  |  |  |  |  |  |  |  | Mtr |  |  |  | 350 |  |  |  |  |  |
| 4 |  |  |  |  | BioMetric with RFID reader Time Attendance & access control System controller at entry point |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 2 |  |  |  |  |  |
| 5 |  |  |  |  | BioMetric with RFID reader Time Attendance System at exit point |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 2 |  |  |  |  |  |
| 6 |  |  |  |  | Time Attendance Software with MS-Access database |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |
| 7 |  |  |  |  | 4 Door 8 Reader Access Control Panel with onboard TCP/IP (master) |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |
| 8 |  |  |  |  | Card reader based door controller |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 8 |  |  |  |  |  |
| 9 |  |  |  |  | ProxCard II Proximity Access Card Non Programmed |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 120 |  |  |  |  |  |
| 10 |  |  |  |  | CARD PRINTING ( One Side) With Card Holder |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 120 |  |  |  |  |  |
| 11 |  |  |  |  | 600lbs electromagnetic lock unmonitored type 12V LED Lamp Only |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 5 |  |  |  |  |  |
| 12 |  |  |  |  | Power Supply Unit – Power through UPS |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 5 |  |  |  |  |  |
| 13 |  |  |  |  | System commissioning charges. |  |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  | 1 |  |  |  |  |  |
| SL NO |  |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  | UNIT |  |  |  | QTY |  |  |  |  |  |
| H- FIRE SYSTEMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ANALOGUE ADDRESSABLE FIRE DETECTION AND ALARM SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | 2 LOOP Analogue intelligent Addressable Fire Alarm Control Panel for addressable devices per loop like SMOKE DETECTOR, HEAT DETECTOR, MULTI DETECTOR, MCP, flasher cum hooter, control modules, response indicators etc. and shall field Programmable and with multiple characters LCD display as per the detailed specification complete with battery backup of 24 hours operation and in 5 minute alarm operation with battery charger and annunciator window etc. System shall have provision to integrate with your existing. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 1 |  |  |  |  |
| 2 |  |  |  |  | Addressable Photo electric smoke detectors with required accessories along with Junction box & Glands & Terminations. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 90 |  |  |  |  |
| 3 |  |  |  |  | Addressable Rate of rise thermal detectors with required accessories along with Junction box & Glands & Terminations. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 5 |  |  |  |  |
| 4 |  |  |  |  | Addressable Multisensor smoke detectors with required accessories along with Junction box & Glands & Terminations. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 55 |  |  |  |  |
| 5 |  |  |  |  | Addressable Manual call points, ABS plastic, square 4 "x 4" , with Break glass message embedded on the glass, enclosed along with the break glass in junction box & Terminations. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 9 |  |  |  |  |
| 6 |  |  |  |  | Addressable strobe light cum sounder. The strobe light cum sounder shall be made of ABS plastic and have the Db level of 110dbs and a multi tone facility. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 10 |  |  |  |  |
| 7 |  |  |  |  | Response indicator with required accessories. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 55 |  |  |  |  |
| 8 |  |  |  |  | SITC of Addressable Fault isolator modules with required accessories including JB & glands |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 7 |  |  |  |  |
| 9 |  |  |  |  | Addressable monitor modules. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 8 |  |  |  |  |
| 10 |  |  |  |  | SITC of Addressable Control modules for activate hooter cum strobe or trip / activate off any device which are related with fire and required accessories including JB & glands |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 2 |  |  |  |  |
| 11 |  |  |  |  | Auto glow exit light. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 10 |  |  |  |  |
| 12 |  |  |  |  | Auto dialer. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 1 |  |  |  |  |
| 13 |  |  |  |  | 2C x 1.5 sq. mm, CU, PVC insulated FRLS Armored Cable with Accessories confirming to IS standards and specifications. (use for loop line and for 24 v dc line) |  |  |  |  |  |  |  |  |  |  | Mtr |  |  |  |  | 1500 |  |  |  |  |
| 14 |  |  |  |  | System commissioning charges. |  |  |  |  |  |  |  |  |  |  | LOT |  |  |  |  | 1 |  |  |  |  |
|  |  |  |  |  | FIRE EXTINGUISHER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  | CO2 Type Fire Extinguisher 4.5 Kg Capacity ISI Mark IS:15683 fitted with wheel type ISI mark wheel type valve and all other necessary fittings, Fire Rating : 21 B. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 5 |  |  |  |  |
| 2 |  |  |  |  | Foam CO2 Type (Cartridge Type) Fire Extinguisher 9 Ltr Capacity ISI Mark IS: 15683 Fire Rating. |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 1 |  |  |  |  |
| 3 |  |  |  |  | ABC POWDER 5KG ISI Mark IS:15683 |  |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  | 15 |  |  |  |  |
| SL NO |  |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  | UNIT |  |  |  |  | QTY |  |  |  |  |
| - AIR CONDITIONING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Supply of Imported Air-cooled Top Discharge Full Inverter VRF SYSTEM Unit with R410a Refrigerant, Air-cooled Condenser along with inbuilt microprocessor panel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | OUTDOOR UNIT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 34 HP |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
|  |  |  |  |  |  | 44 HP |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 2 |  |  |  |  |  | VRF Indoor Units :- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | High Wall Mounted Split units. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a |  |  |  |  |  | 0.8 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 3 |  |  |  |  |  |
| b |  |  |  |  |  | 1 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 15 |  |  |  |  |  |
| c |  |  |  |  |  | 1.28 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 3 |  |  |  |  |  |
| d |  |  |  |  |  | 1.6 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| e |  |  |  |  |  | 2.03 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| f |  |  |  |  |  | 2.28 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  | 4 Way Cassette Units. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a |  |  |  |  |  | 1.28 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| b |  |  |  |  |  | 1.6 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 7 |  |  |  |  |  |
| c |  |  |  |  |  | 2.03 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 13 |  |  |  |  |  |
| d |  |  |  |  |  | 2.57 TR |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 12 |  |  |  |  |  |
| 3 |  |  |  |  |  | Refrigerant Joints for Indoor Units |  |  |  |  |  |  |  |  |  | Nos. |  |  |  |  |  |  |  | 54 |  |  |  |  |  |
| B |  |  |  |  |  | Price for Supply Installation & Commissioning of Ancillary Items for VRF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | Installation & Commissioning of VRF Units , with outdoor mounting, pressure testing, nitrogen flushing etc |  |  |  |  |  |  |  |  |  | Lot |  |  |  |  |  |  |  | 3 |  |  |  |  |  |
| 2 |  |  |  |  |  | Refrigerant Gas ( R - 410a ) VRF |  |  |  |  |  |  |  |  |  | Kg |  |  |  |  |  |  |  | 60 |  |  |  |  |  |
| 3 |  |  |  |  |  | V I Pads- VRF |  |  |  |  |  |  |  |  |  | Lot |  |  |  |  |  |  |  | 10 |  |  |  |  |  |
| 4 |  |  |  |  |  | Installation of VRF Indoor Units |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |  |  | 57 |  |  |  |  |  |
| 5 |  |  |  |  |  | Refrigerant Piping with insulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Suction/ Liquid line with Nitrile rubber insulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 11/2" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 60 |  |  |  |  |  |
|  |  |  |  |  |  | 11/4" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 15 |  |  |  |  |  |
|  |  |  |  |  |  | 11/8" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 65 |  |  |  |  |  |
|  |  |  |  |  |  | 7/8 " OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 65 |  |  |  |  |  |
|  |  |  |  |  |  | 3/4 " OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 85 |  |  |  |  |  |
|  |  |  |  |  |  | 5/8" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 130 |  |  |  |  |  |
|  |  |  |  |  |  | 1/2" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 150 |  |  |  |  |  |
|  |  |  |  |  |  | 3/8" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 170 |  |  |  |  |  |
|  |  |  |  |  |  | 1/4" OD |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 110 |  |  |  |  |  |
| 6 |  |  |  |  |  | Metallic Drain piping 32 mm with insulation to avoid RAT BITE All complete |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 400 |  |  |  |  |  |
| 7 |  |  |  |  |  | Electricals/Cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Control Cable 2C x 1.5 sqm for VRF unit |  |  |  |  |  |  |  |  |  | Rmt |  |  |  |  |  |  |  | 500 |  |  |  |  |  |
| J - FURNITURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sl.No. |  |  |  |  |  |  | ITEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QTY |  |  |  |  |  |
|  |  |  |  |  |  |  | OFFICERS CABIN-07 NOS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  | Main Desk with MAIN TOP- MDF + VENEER + PU Coating, Size :1800 W * 900 D * 750 H TOP THICKNESS — 65 mm, MODESTY- Size : 1640 X 600 X16 MM, MDF+ Veneer + PU Coating, MOBILE PEDESTAL- MDF + Veneer + PU Coating, Size :510W * 635H * 445D mm, [RU TOP- MDF + VENEER + PU Coating, Size :1200 W x 445 D x 660 HTOP THICKNESS — 25 mm (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |
| 2 |  |  |  |  |  |  | High Back Chair Seat & back made up of 1.2 cm thick hot pressed plywood upholstered with fabric & moulded polyurethane foam having density of 45 +/-2 kg/m3 & hardness 20 +/-2 with the back size 48 cm W x 74 cm H & seat size 50 cm W x 49.5cm H having fixed type armrest with top made of moulded polyurethane & mounted on fixed MS tubular arm rest support chrome plated, static vertical arm supported adjustment of +/- 1.5 cm. Front pivot synchro mechanism having 360 degree revolving type, single point control, front pivot with tilt fit resting on ground, tilt tension adjustment, 4 position locking with anti-shock feature & the seat back tilting ratio 1: 2 (11 degree seat tilt, Ott 22 degree back tilt) with back rest pneumatic height adjustment of 10 cm & the legs is fabricated from CR steel, Chrome plated with black polypropylene hub cap and 5 nos. twin wheel castors (castor wheel dia. 5.0 cm). The pedestal is 65.0cm. Pitch - center dia. (75.0 cm with castors). (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |
| 3 |  |  |  |  |  |  | Visitor Chair, SEAT/BACK ASSEMBLY: The seat is made up of 1.2 cm. thick hot pressed plywood upholstered with fabric and moulded Polyurethane Foam. The back is made up 1.2 cm. thick hot pressed plywood upholstered with replaceable fabric upholstery covers and moulded polyurethane foam. The back ply and foam is designed - with contoured lumber support for comfortable seating posture. MID BACK SIZE 48.0cm.(W) X 64.5cm.(H), SEAT SIZE 51.0cm.(W) X 48.0cm.(D). POLYURETHANE FOAM: The polyurethane foam for seat and back is moulded with density = 45±2 kg/m3 and Hardness = 20±2, ARMRESTS: The armrest top is made of moulded polyurethane (PU) and mounted on to a fixed type M.S. tubular armrest support chrome plated. The arm support has static vertical adjustment of ±1.5cm. FIXED BACKREST: The backrest consists of a fixed type mechanism i.e no back up/down adjustment, LEG FRAME ASSEMBLY: Leg frame welded assembly made from 35 X 15 X 16BG (1.6THK) Elliptical M.S ERW tube with base plate chrome plated for seat fixing (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 21 |  |  |  |  |  |
| 4 |  |  |  |  |  |  | Sliding Door Unit - Sliding door unit frame type constructed for rigidity with top having sliding door plastic roller with ball bearing 02 adjustable shelves fitted with lock made of spray phosphated before painting overall dimension 1168 H X 450D X 900W with one shelves (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |
|  |  |  |  |  |  |  | CABINS-13 NOS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  | Table- Pedestal Table of size 1500 x 750 x 740 ,Top & Side Panel made of top of 25 mm thick plain particle board clad with 0.6mm thick post formed Laminate and 1 mm Thick Backing Laminate Flat Edge duly sealed with 2 mm thick PVC beading Modesty panel of 18mm thick plain particle Board clad with 1.0 mm thick DL on both sides, Edge with 2 mm thick PVC beading, [RU of size 1050 x 520 x 705 & Three Drawers Pedestal (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |  |  |  |  |
| 6 |  |  |  |  |  |  | Mid Back - Chair Seat & back made up of 1.2 cm thick hot pressed plywood upholstered with fabric & moulded polyurethane foam having density of 45 +/-2 kg/m3 & hardness 20 +/-2 with the back size 48 cm W x 66 cm H & seat size 50 cm W x 49.5cm H having fixed type armrest with top made of moulded polyurethane & mounted on fixed MS tubular arm rest support chrome plated, static vertical arm supported adjustment of +/- 1.5 cm. Front pivot synchro mechanism having 360 degree revolving type, single point control, front pivot with tilt fit resting on ground, tilt tension adjustment, 4 position locking with anti-shock feature & the seat back tilting ratio 1: 2 (11 degree seat tilt, 22 degree back tilt) with back rest pneumatic height adjustment of 10 cm & the legs is fabricated from CR steel, Chrome plated with black polypropylene hub cap and 5 nos. twin wheel castors (castor wheel dia. 5.0 cm). The pedestal is 65.0cm. Pitch - center dia. (75.0 cm with castors) (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |  |  |  |  |
| 7 |  |  |  |  |  |  | Visitor Chair - The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with contoured lumbar support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE -47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co-polymer Polypropylene. The powder coated tubular frame is cantilever type & made of dia. 25.4mm x 2mm thick M.S. ER.W. Tube. (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26 |  |  |  |  |  |
| 8 |  |  |  |  |  |  | Sliding Door Unit - Sliding door unit frame type constructed for rigidity with top having sliding door plastic roller with ball bearing 02 adjustable shelves fitted with lock made of spray phosphated before painting overall dimension 1168 H X 450D X 900W with one shelves (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 13 |  |  |  |  |  |
|  |  |  |  |  |  |  | DIRECTOR'S CABIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  | Main Desk with ERU & Mobile Pedestal – Executive Table of size 2200 x 1190 x 750 made of 30 mm thick pre laminated board top , side panel of 44mm thick pre laminated board Modesty- 18mm PL Board, ERU with drawer of size 1380 x 480 x 700 having 25mm thick ply board & 18mm thick ply board, Mobile Pedestal of size 406 x 550 x 640 having 25mm thick ply top (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 10 |  |  |  |  |  |  | Mid Back Chair with Head Rest - Moulds itself to your back, provides adequate back support, and makes the long hours of seating comfortable. The net also provides opportunity for sufficient air circulation and helps in preventing heat accumulation. Adjustable lumber support - Supports the back and suits the requirement of maximum number of people and makes the long hours of seating a pleasure Adjustable headrest - Supports the head properly to suit the requirement of maximum users and give a comfortable feeling from head to lumber, Aluminium metal 5 pronged pedestals – Metallic pedestal has high load bearing capacity and sleek looks enhancing the aesthetics of the chair. Multi position locking with anti-shock feature. - Allows for the user to sit with maximum comfort and ease for long hours. Height 118-149 cm, seat height 46-54.5 cm width-72.5cm, depth 72.5 cm (Godrej & Equivalent make ) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 11 |  |  |  |  |  |  | Visitor Chair - SEAT/BACK ASSEMBLY: The seat is made up of 1.2 cm. thick hot pressed plywood upholstered with fabric and moulded Polyurethane Foam. The back is made up 1.2 cm. thick hot pressed plywood upholstered with replaceable fabric upholstery covers and moulded polyurethane foam. The back ply and foam is designed with contoured lumber support for comfortable seating posture. MID BACK SIZE 48.0cm.(W) X 64.5cm.(H), SEAT SIZE 51.0cm.(W) X 48.0cm.(D). POLYURETHANE FOAM: The polyurethane foam for seat and back is moulded with density = 45±2 kg/m3 and Hardness = 20±2, ARMRESTS: The armrest top is made of moulded polyurethane (P.U) and mounted on to a fixed type M.S. tubular armrest support chrome plated The arm support has static vertical adjustment of ±1.5cm. FIXED BACKREST: The backrest consists of a fixed type mechanism i.e., no back up/down adjustment, LEG FRAME ASSEMBLY: Leg frame welded assembly made from 35 X 15 X 16BG (1.6THK) Elliptical M.S ERW tube with base plate chrome plated for seat fixing (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |
|  |  |  |  |  |  |  | MD CHAMBER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | Main Desk with ERU, Joining Table, Back Unit & Mobile Pedestal, Table & ERU & Top Post Formed made of 25 mm MFC (Melamine Faced Chipboard), plus 30 mm MDF (Medium Density" Fiberboard), 18mm MFC Modesty Panel, Legs- 25 mm MFC, plus 30 mm MDF right and left pad. Main Desk size 1800 x 900 x 750, ERU of size 1200 X 600 X 730 with drawers unit & one Mobile drawer unit of size 400 x 560 x560 for main desk, Joining desk of size 1200 X 1050 X 750, Back Unit Top made of 25 mm MFC (Melamine Faced Chipboard), Door & other panel -18 mm MFC. The Dimension of Back Unit is 1400 x 470 x 760 mm. (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 13 |  |  |  |  |  |  | High Back Chair - Revolving Chair sharp & elegant design with reach leather finish ergonomically design with natural leather upholstery at all contact areas Knee tilt synchronous mechanism with four position locking & anti shock feature, height adjustable arm rest, Height-113.5cm, W-62cm, D-60cm, Seat Height- 42.5 cm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 14 |  |  |  |  |  |  | Visitor Chair - SEAT/BACK ASSEMBLY: The seat is made up of 1.2 cm. thick hot pressed plywood upholstered with fabric and moulded Polyurethane Foam. The back is made up 1.2 cm. thick hot pressed plywood upholstered with replaceable fabric upholstery covers and moulded polyurethane foam. The back ply and foam is designed with contoured lumber support for comfortable seating posture. MID BACK SIZE 48.0cm.(W) X 64.5cm.(H), SEAT SIZE 51.0cm.(W) X 48.0cm.(D). POLYURETHANE FOAM: The polyurethane foam for seat and back is moulded with density --, 45±2 kg/m3 and Hardness = 20±2, ARMRESTS: The armrest top is made of moulded polyurethane (P.U) and mounted on to a fixed type M.S. tubular armrest support chrome plated. The arm support has static vertical adjustment of ±1.5cm. FIXED BACKREST: The backrest consists of a fixed type mechanism i.e., no back up/down adjustment, LEG FRAME ASSEMBLY: Leg frame welded assembly made from 35 X 15 X 16BG (1.6THK) Elliptical M.S ERW tube with base plate chrome plated for seat fixing (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |
| 15 |  |  |  |  |  |  | Table - Size 3000W x 1200 D with laminate top with the vertical & horizontal wire management across length & height of the Table access flop mounted on table top for ease of use. All steel components have powder coated paint Anticorrosion treated components, oven baked powder coating process (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 16 |  |  |  |  |  |  | Mid Back Chair with Head Rest - Moulds itself to your back, provides adequate back support, and makes the long hours of seating comfortable. The net also provides opportunity for sufficient air circulation and helps in preventing heat accumulation. Adjustable lumber support - Supports the back and suits the requirement of maximum number of people and makes the long hours of seating a pleasure Adjustable headrest - Supports the head properly to suit the requirement of maximum users and give a comfortable feeling from head to lumber, Aluminium metal 5 pronged pedestals – Metallic pedestal has high load bearing capacity and sleek looks enhancing the aesthetics of the chair. Multi position locking with anti-shock feature. Allows for the user to sit with maximum comfort and ease for long hours. Height 118-149 cm, seat height 46-54.5 cm width-72.5cm, depth 72.5 cm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 17 |  |  |  |  |  |  | Mid Back Chair - Moulds itself to your back, provides adequate back support, and makes the long hours of seating comfortable. The net also provides opportunity for sufficient air circulation and helps in preventing heat accumulation. Adjustable lumber support - Supports the back and suits the requirement of maximum number of people and makes the long hours of seating a pleasure Adjustable headrest - Supports the head properly to suit the requirement of maximum users and give a comfortable feeling from head to lumber, Aluminium metal 5 pronged pedestals – Metallic pedestal has high load bearing capacity and sleek looks enhancing the aesthetics of the chair. Multi position locking with anti-shock feature. Allows for the user to sit with maximum comfort and ease for long hours. Height 118-149 cm, seat height 46-54.5 cm width-72.5cm, depth 72.5 cm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |
| 18 |  |  |  |  |  |  | Three Seater Sofa, of Frame: Pine wood with moisture content within 10%, Ply wood of IS 303 commercial plywood Upholstery: PVC in Black shade of 1 mm thickness and 525 GSM. Foam: Moulded foam of density 48 grams /cubic cm Pocket Spring: 49 nos. of thickness 2 mm Legs: PVC legs of 4 inches. Dimension of Three Seater -1900W x 900D x 860H mm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 19 |  |  |  |  |  |  | Single Seater Sofa, of Frame: Pine wood with moisture content within 10%, Ply wood of IS 303 commercial plywood Upholstery: PVC in Black shade of 1 mm thickness and 525 GSM. Foam: Moulded foam of density 48 grams /cubic cm Pocket Spring: 49 nos. of thickness 2 mm Legs: PVC legs of 4 inches. Dimension of Single Seater -840W x 910D x 860Hmm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 20 |  |  |  |  |  |  | Coffee Table Coffee Table made of 8mm tempered glass Top MDF understructure with PVC covering Pu paint strip on glass top with silver powder Glass top attached to the understructure via UV disc Load bearing capacity :30 Kg, Dimension -1000Wx 600Dx 450H (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 21 |  |  |  |  |  |  | Corner Table made of 8mm tempered glass top MDF understructure with PVC covering Pu paint strip on glass top with silver powder Glass top attached to the understructure via UV disc Load bearing capacity : 20, Dimension -600Wx 600Dx 540H Kg (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | PA OF M.D. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  | Main Desk of size 1350 x 750 with ERU of size 1200 x 600 & 01 No. Mobile Pedestal Table made of 25mm thick PLB with PVC lipping Top of size 1350(W)x750(D)x750(H) & side panels & modesty panel made of 18mm thick PLB with PVC lipping. One free standing Mobile pedestal of size 390 x 435 x 529 having one box & one filling drawer made of 18mm thick PLB fronts & top. Side Unit of size 1200 x 600 with top made of 25mm thick PLB with PVC lipping & side panels & Modesty Panels made of 18mm thick PLB with PVC lipping (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 23 |  |  |  |  |  |  | Mid Back Chair The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE - 47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co- polymer Polypropylene. 4) CENTER TILT SYNCHRO MECHANISM. PNEUMATIC HEIGHT ADJUSTMENT -The pneumatic height adjustment has an adjustment stroke of 12.0 cm. TELESCOPIC BELLOW ASSEMBLY: The bellow is 3 piece telescopic type and injection moulded in black Polypropylene. PEDESTAL ASSEMBLY: The pedestal is injection moulded in black 30% glass -filled Nylon and fitted with 5 nos. twin wheel castors. The pedestal is 66.0cm pitch -center dia. (76.0 cm with castors.) TWIN WHEEL CASTORS: The twin wheel castors are injection moulded in Black Nylon (Godrej or Equivalent) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |
| 24 |  |  |  |  |  |  | Visitor Chair The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with contoured lumbar support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE - 47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co-polymer Polypropylene. The powder coated tubular frame is cantilever type & made of dia 25.4mm x 2mm thk M.S. ER.W. Tube. (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 25 |  |  |  |  |  |  | Sliding Door Unit - Sliding door unit frame type constructed for rigidity with top having sliding door plastic roller with ball bearing 02 adjustable shelves fitted with lock made of spray phosphated before painting overall dimension 1168 H X 4500 X 900W with one shelves (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 26 |  |  |  |  |  |  | Lounge Three Seater Chair - Made of metallic understructure with powder coated with individual flexi back with colorful upholstery, Glide screw at the bottom of level adjustment in case of uneven surface. Dimension 164.0cmL x 70.0cmD x 78.5cmH, Seat Height 43.5cm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | WAITING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  | Three Seater Chair - The LH/RH side frame fitted to the two ends of the ST/BK mounting frame to form the leg assembly. Made of SS J4 Grade tube dia 4.44 cm. (1.75") x 14 BG. Thick. The ST/BK mounting frame assy. Holds the two side frames together. The mounting frames, 2 nos. to connect the side frames. The mounting frame made of MS ERW tube dia. 5.08cm (2") x 10BG thick black painted. The ST/BK mounted on 14 BG thick 2cm x 4 cm x 4 cm Recta tube which welded on the beam of ST/BK mounting frame. The ST/BK assembly consists of 12mm thick plywood insert with Polyurethane foam having density = 45 +/-2 Kg./cm3 and hardness of the P.U. foam = 18 to 22 Kg. The complete moulded ST/BK assembly covered with a replaceable fabric upholstery cover. SEAT/BACK SIZE : 54.5CM (w) X 46.7CM (d) X 12.5CM (T) The sid frames are fitted with front and rear bottom shoes made of injection moulded polypropylene. The sofa upholstered with stain repellant 'Velvetine plus' fabric. Dimension- 1830 W x 783D x 750Hmm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 28 |  |  |  |  |  |  | Two Seater Chair - The LH/RH side frame fitted to the two ends of the ST/BK mounting frame to form the leg assembly. Made of SS J4 Grade tube dia 4.44 cm. (1.75") x 14 BG. Thick. The ST/BK mounting frame assy. Holds the two side frames together. The mounting frames, 2 nos. to connect the side frames. The mounting frame made of MS ERW tube dia. 5.08cm (2") x 10BG thick black painted. The ST/BK mounted on 14 BG thick 2cm x 4 cm x 4 cm Recta tube which welded on the beam of ST/BK mounting frame. The ST/BK assembly consists of 12mm thick plywood insert with Polyurethane foam having density = 45 +/-2 Kg./cm3 and hardness of the P.U. foam = 18 to 22 Kg. The complete moulded ST/BK assembly covered with a replaceable fabric upholstery cover. SEAT/BACK SIZE : 54.5CM (w) X 46.7CM (d) X 12.5CM (T) The side frames are fitted with front and rear bottom shoes made of injection moulded polypropylene. The sofa upholstered with stain repellant 'Velvetine plus' fabric. Dimension- 1257 W x 775D x 745Hmmmm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |
| 29 |  |  |  |  |  |  | Corner Table - of size 600W x 600D x 450H made up of 8 mm tempered glass Top, 6 mm tempered bottom glass for storage option, Silver powder coated understructure, Glass glass attached to the understructure via UV disc (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 30 |  |  |  |  |  |  | Reception Table double Arch with DL Top Modular elegant table of curvilinear decorative laminate top 18mm thick at 725mm H (Approx.) and l0 mm frosted glass at 1035mm H(Approx.) above the work surface. The understructure made of mild steel tube dia. 50.8mm. The front modesty panel below and above work surface made of 0.8mm mild steel perforated sheet. nova 3 drawers free standing pedestal of size 300W x 435Dx 646H(Approx.) (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 31 |  |  |  |  |  |  | Mid Back Chair - The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with contoured lumbar support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE - 47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co-polymer Polypropylene. 4) CENTER TILT SYNCHRO MECHANISM. PNEUMATIC HEIGHT ADJUSTMENT -The pneumatic height adjustment has an adjustment stroke of 12.0 cm. ELESCOPIC BELLOW ASSEMBLY: The bellow is 3 piece telescopic type and injection moulded in black Polypropylene. PEDESTAL ASSEMBLY: The pedestal is injection moulded in black 30% glass -filled Nylon and fitted with 5 nos. twin wheel castors. The pedestal is 66.0cm pitch -center dia. (76.0 cm with castors.) TWIN WHEEL CASTORS: The twin wheel castors are injection moulded in Black Nylon (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 32 |  |  |  |  |  |  | Main Desk of size 1350 x 750 with ERU of size 1200 x 600 & 01 No. Mobile Pedestal Table made of 25mm thick PLB with PVC lipping Top of size 1350(W)x750(D)x750(H) & side panels & modesty panel made of 18mm thick PLB with PVC lipping. One free standing Mobile pedestal of size 390 x 435 x 529 having one box & one filling drawer made of 18mm thick PLB fronts & top. Side Unit of size 1200 x 600 with top made of 25mm thick PLB with PVC lipping & side panels & Modesty Panels made of 18mm thick PLB with PVC lipping (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 33 |  |  |  |  |  |  | Mid Back Chair - The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with contoured lumbar support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE -47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co-polymer Polypropylene. 4) CENTER TILT SYNCHRO MECHANISM. PNEUMATIC HEIGHT ADJUSTMENT -The pneumatic height adjustment has an adjustment stroke of 12.0 cm. TELESCOPIC BELLOW ASSEMBLY: The bellow is 3 piece telescopic type and injection moulded in black Polypropylene. PEDESTAL ASSEMBLY: The pedestal is injection moulded in black 30% glass -filled Nylon and fitted with 5 nos. twin wheel castors. The pedestal is 66.0cm pitch -center dia. (76.0 cm with castors.) TWIN WHEEL CASTORS: The twin wheel castors are injection moulded in Black Nylon (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | CONFERENCE ROOM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  | Conference Table for 32 Persons - Top thickness 31.6 (18mm + 12mm + 0.6 mm DL(both sides + 0.4mm Membrane) Edge profile Waterfall Edge l0mm radius on top edge and 5mm at bottom. Legs- Made from 25mm PPB having a straight profile with half round edges and clad with 0.6mm thick Post Forming laminate. Overall thickness of leg is 26.2mm. Modesty Panel- Made from PLT (Prelaminated Twin) boards of 18mm thick. Skin PVC Membrane foil (0.4mm thick) clad on the substrate MDF using PU glue for better adhesion. This foil is pre -coated with layer of polyurethane for better scratch resistance. Single Seater 760 W x 600 D, Two Seater 1360 W x 600 D, Half Round (2 Seater)- R 713, Quarter Round (1 Seater)- R 713 (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 35 |  |  |  |  |  |  | High Back Chair - Revolving Chair sharp & elegant design with reach leather finish ergonomically design with natural leather up upholstery at all contact areas Knee tilt synchronous mechanism with four position locking & anti shock feature, height adjustable arm rest, Height-113.5cm, W-62cm, D-60cm, Seat Height- 42.5 cm (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 36 |  |  |  |  |  |  | Chair - Seat & back made up of 1.2 cm thick hot pressed plywood upholstered with fabric & moulded polyurethane foam having density of 45 +/-2 kg/m3 & hardness 20 +/-2 with the back size 48 cm W x 66 cm H & seat size 50 cm W x 49.5c having fixed type armrest with top made of moulded polyurethane & mounted on fixed MS tubular arm rest support chrome plated, static vertical arm supported adjustment of +/- 1.5 cm. Front pivot synchro mechanism having 360 degree revolving type, single point control, front pivot with tilt fit resting on ground, tilt tension adjustment, 4 position locking with anti-shock feature & the seat back tilting ratio 1: 2 (11 degree seat tilt, 22 degree back tilt) with back rest pneumatic height adjustment of 10 cm & the legs is fabricated from CR steel, Chrome plated with black polypropylene hub cap and 5 nos. twin wheel castors (castor wheel dia. 5.0 cm). The pedestal is 65.0cm. Pitch - center dia. (75.0 cm with castors). (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |  |  |  |  |
| 37 |  |  |  |  |  |  | Chair - SEAT/BACK ASSEMBLY: The seat is made up of 1.2 cm. thick hot pressed plywood upholstered with fabric and moulded Polyurethane Foam. The back is made up 1.2 cm. thick hot pressed plywood upholstered with replaceable fabric upholstery covers and moulded polyurethane foam. The back ply and foam is designed with contoured lumber support for comfortable seating posture. MID BACK SIZE 48.0cm.(W) X 64.5cm.(H), SEAT SIZE 51.0cm.(W) X 48.0cm.(D). POLYURETHANE FOAM: The polyurethane foam for seat and back is moulded with density = 45±2 kg/m3 and Hardness = 20±2, ARMRESTS: The armrest top is made of moulded polyurethane (PU) an mounted on to a fixed type M.S. tubular armrest support chrome plated. The arm support has static vertical adjustment of ±1.5cm. FIXED BACKREST: The backrest consists of a fixed type mechanism i.e., no back up/down adjustment, LEG FRAME ASSEMBLY: Leg frame welded assembly made from 35 X 15 X 16BG (1.6THK) Elliptical M.S ERW tube with base plate chrome plated for seat fixing (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 39 |  |  |  |  |  |
|  |  |  |  |  |  |  | OPTIMIZER STORAGE SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  | Optimizer Storage system with Drive D-2 Type Two Bay on 12' Rail length (Each Unit Depth 457mm, Height-1980mm) Drive Type storage system on rail (fixed and movable) bolted to undercarriage which rolls on to the channels and embedded to the ground. Cabinet made out of 0.8 mm thick CRCA Steel Sheet of IS: 513 Grade D consists of 1 main unit & then add On Units each Unit with 5 Loading Levels. Height from ground 2080mm (1980 body + 65mm undercarraige+35 Channels system). Cabinet with antirust treatment & powder coated paint oven baked 180 degree to 200 degree C load bearing 60 Kg. UDL per shelf. Movement of units by mechanically through PU Drive drive wheel and sprocket -chain Tensioner with centralized locking (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 39 |  |  |  |  |  |  | Optimizer Storage system with Drive D-4 Type 4 bay on 15' Rail length (Each Unit Depth 457mm, Height-1980mm) Drive Type storage system on rail (fixed and movable) bolted to undercarriage which rolls on to the channels and embedded to the ground. Cabinet made out of 0.8 mm thick CRCA Steel Sheet of IS: 513 Grade D consists of 1 main unit & then add On Units each Unit with 5 Loading Levels. Height from ground 2080mm (1980 body + 65mm undercarraige+35 Channels system). Cabinet with antirust treatment & powder coated paint oven baked 180 degree to 200 degree C load bearing 60 Kg. UDL per shelf. Movement of units by mechanically through PU Drive drive wheel and sprocket -chain Tensioner with centralized locking (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | STORE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  | Steel MF System of size 6'x36"x18" with Six Shelves made of slotted angle Prime quality CRC sheet conforming to IS 513D/10748/5986. The pitches of the slots at 19.05mm, Panels/Shelves: 6 Bend bending process of 905(L) x 448(D) of load capacity 80 Kgs. UDL Max. per shelf , All fasteners of electro perk arised as per IS : 3655 -1985 Painting: All Steel components of powder coated as per IS: 101/1964 (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |
|  |  |  |  |  |  |  | WORKSTATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  | Open Plan Office System for 112 Persons with Partition height of 1200mm, Recta Work surface of size 1200 x 600, Penta Work surface of size 1500 x 1500. Each work surface with 01 No. 3 Drawers Pedestal, 01 No. KBPT & 01 No. CPU Trolley PANEL THICKNESS - 56 MM, HT. 1200 MM( APPROX), TILE CONFIGURATION FOR ALL PANELS :- ABOVE MAG FAB TILE & BELOW TILE, RACEWAY - FOR MOUNTING OF SWITCHES: 150MM METAL RACEWAY AT SKIRTING LEVEL FOR ALL PANELS, WORKSURFACE: IN PRELAMINATED PARTICLE BOARD AND PVC LIPPING; BOARD THK. - 25MM (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 42 |  |  |  |  |  |  | Mid Back Chair - The seat and back are made up of 1.2 cm. thick hot -pressed plywood, upholstered with fabric upholstery covers and moulded Polyurethane foam. The back foam is designed with contoured lumbar support for extra comfort. The seat has extra thick foam on front edge to give comfort to popliteal area. Dimension BACK SIZE - 47.5 cm. (W) x 58.0cm. (H) & SEAT SIZE 47.0 cm. (W) x 48.0 cm. (D). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kg/m3 and Hardness = 20 +/- 2 at 25% compression. 3) ARMRESTS: The one-piece armrests are injection moulded from black Co-polymer Polypropylene. 4) CENTER TILT SYNCHRO MECHANISM. PNEUMATIC HEIGHT ADJUSTMENT -The pneumatic height adjustment has an adjustment stroke of 12.0 cm. TELESCOPIC BELLOW ASSEMBLY: The bellow is 3 piece telescopic type and injection moulded in black Polypropylene. PEDESTAL ASSEMBLY: The pedestal is injection moulded in black 30% glass -filled Nylon and fitted with 5 nos. twin wheel castors. The pedestal is 66.0cm pitch -center dia. (76.0 cm with castors.) TWIN WHEEL CASTORS: The twin wheel castors are injection moulded in Black Nylon (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 |  |  |  |  |  |
| 43 |  |  |  |  |  |  | Chair - SEAT/BACK ASSEMBLY: The seat and back are made up of 1.2 cm thick hot pressed plywood, upholstered with fabric and moulded Polyurethane foam with PVC lipping all around. The back foam is designed with contoured lumbar support for extra comfort. SIZE: 41.0cm. (W) X 46.0cm. (H). POLYURETHANE FOAM: The Polyurethane foam is moulded with density = 45 +/-2 kgs. / M3 and Hardness = 20 +/-2 on Hampden machine at 25% compression. ARMRESTS- The armrest is made of black integral skin polyurethane with 50-70 Shore A hardness and reinforced with MS insert. The P.U. armrest is fixed to black powder coated armrest bracket made of 0.5cm thick. H.R. Steel. PEDESTAL ASSEMBLY: The pedestal is fabricated from 0.2cm thick CR steel, powder coated and fitted with an injection moulded black Polypropylene hub cap and 5 nos. twin wheel castors.(castor wheel dia. 5.0cm.)The pedestal is 55.0cm. Pitch -center dia. (65.0 cm with castors). (Godrej & Equivalent make) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |
| K - Audio Visual System for Conference Hall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No |  |  |  |  |  |  |  |  | Description |  |  |  | Make | UoM |  |  |  |  |  |  |  |  | Qty |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  | 4.0K lm, WXGA projector |  |  |  | Panasonic / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  | Ceiling mount kit for projector |  |  |  | Online / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  | 109" , 16:10 Aspect Ratio , Motorized Projection Screen |  |  |  | Milan / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | 65" Professional LED Display |  |  |  | Panasonic / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  | 43"LED display |  |  |  | Panasonic / equivalent | Piece |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  | Passive Face Plate with VGA + Audio and HDMI |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  | 3 Input Switcher with DTP Out |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  | Under mount |  |  |  | Extron/ equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  | Presentation switcher scaler |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  | HDMI to four Output DTP DA - 230 feet (70 m) |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  | HDMI Patch Cables , 6 Feet |  |  |  | Kramer / equivalent | Piece |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  | VGA + Audio Patch Cables , 6 Feet |  |  |  | Kramer / equivalent | Piece |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  | HDMI Tx |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  | HDMI Receiver |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |
| Display And video- Switching System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No |  |  |  |  |  |  |  |  | Description |  |  |  | Make | UoM |  |  |  |  |  |  |  |  | Qty |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  | Group 500 HD codec, EagleEyeIV-12x camera, mic array, univ. |  |  |  | Polycom / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  | Partner Advantage, One Year, Real Presence Group 500-720p: |  |  |  | Polycom / equivalent | Service |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  | Group Series Multipoint License - 6-way on Group 500 or 8-way |  |  |  | Polycom / equivalent | License |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  | Group Series 1080p HD License-1080 encode/decode for people & |  |  |  | Polycom / equivalent | License |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  | Camera wall mount |  |  |  | NT | Set |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| Audio System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No |  |  |  |  |  |  |  |  | Description |  |  |  | Make | UoM |  |  |  |  |  |  |  |  | Qty |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  | Gooseneck Microphone |  |  |  | Beyerdynamic / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  | Shock Mount |  |  |  | Beyerdynamic / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  | Wireless handheld microphone |  |  |  | Beyerdynamic / equivalent | Piece |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  | DSP |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  | Wall Mount Speakers |  |  |  | Extron / equivalent | Pair |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  | Audio Power Amplifier |  |  |  | Extron / equivalent | Piece |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  | Cable,Connectors and AV Rack as Per actual |  |  |  | Belden,Kramer,Dlink and Valrack | Meter |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Installation, Cabling, Termination and coordination with other , Programming Charges |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L- Fittings of Water Purifier & Water Cooler |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No |  |  |  |  |  |  |  |  | Description |  |  |  | Make | UoM |  |  |  |  |  |  |  |  | Qty. |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  | Supply & Installation of water purifier with 500 liters capacity per day. All complete work. |  |  |  | Livpro or Equivalent | Each |  |  |  |  |  |  |  |  | 01 |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  | Supply and installation of water cooler. The water cooler should have two or more taps. The water cooler should be connected with the above water purifier. All complete work. |  |  |  | Voltas or Equivalent | Each |  |  |  |  |  |  |  |  | 01 |  |  |  |  |  |